Reg.No. _____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Code:	12EI224/11EI202				      	     					   Time : 3 hours

Subject Title :	DIGITAL SIGNAL PROCESSING				     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the advantages of digital signal processing over analog signal processing.

2.	A radix 2, 512 point DFT has ____ additions and   ______ multiplications.

3.    List the methods of IIR filter design.

4.	Write the relation between analog and digital frequency in Bilinear Transformation Method.

5.	What is Gibbs phenomenon?

6.    Mention the windows used in design of FIR filters.

7.	Give the advantages of Harvard architecture.

8.    What is the difference between SARAM and DARAM?

9.	What is the function of MAC instruction?

10.  Which addressing mode is suitable for FFT computation?

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate DIT and DIF algorithms.

12. 	For the analog transfer function H(s) =  , determine H(z) using impulse

invariant transformation. Assume T= 1 sec.

13. 	Using minimum number of multipliers, realize the cascade structure of

H(z) = 15+12z−1+15z−217+14z−1+17z−2

14. 	How does pipelining architecture affect the operation of digital signal processor?

15. 	Bring out the merits of CISC and RISC architecture.

PART – C  (5 x 15 = 75 MARKS)

16. a. With a neat block diagram, explain the functional elements of a digital signal processing

system.																			(8)

b. Perform circular convolution of x1[n] = {1, 2, 3} and x2[n] = {2,1}							(7)

(OR)

17. 	Perform the linear convolution of the following sequence by overlap add method for the following sequences x[n] = {1, 2, -1, -2, 2, 1, 3, 2} and h[n] = {2, 1,}.

18. 	An 8-point sequence is given by x [n] = {1, 2, 1, 3, 1, -1, 2, 1}.Compute 8-point DFT of x(n) by radix-2 DIT FFT.

(OR)

19. 	a.State and prove the periodicity property of DFT.										(7)

b. Find the 4 point DFT of x[n] = {1, 0, 1, 1}											(8)

20.	 Obtain the direct form I, II and cascade realizations of the LTI system governed by the equation

y[n]=− 34y[n−1]+12y[n−2]+14y[n−3]+x[n]+4x[n−1]+3x[n−2]

(OR)

21. Design a digital Butterworth filter satisfying the following requirements using bilinear transformation technique.

0.707≤ H(ejω)≤ 1; 0≤ω≤π2 H(ejω) ≤ 0.2; 3π4 ≤ ω ≤ π.

22.  Design a linear phase FIR low pass filter with a cut-off frequency c1 = 0.2π radians/sample using rectangular window by taking 7 samples of the window sequence.

(OR)

23. Determine the co-efficient of a linear phase FIR filter of length N = 11 which has a frequency response

H (2k/11) = 1   ; for k=0, 1, 2, 3

0    ; for k=4, 5

24. Draw the internal architecture of TMS320C5x processor and explain the function of each block.

(OR)

25. What are the four phases of an  instruction cycle? Illustrate how pipelining increases the throughput

efficiency of the processor.