Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec 2016

Subject Title :	BASIC PROCESS EQUIPMENT DESIGN			      	     Time : 3 hours

Subject Code:	12BT241								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Stainless steel 304 means __________.

2. 	Give the process equipment symbols for at least minimum two.

3. 	Which is the most suitable valve for irrigation purpose?

4.  	Venturi meter coefficient discharge is __________.

5.  	List out flow pattern in heat exchanger.

6. 	Write down the units of heat flux, heat transfer coefficient?

7. 	What is the relationship between reflux ratio and theoretical stage?

8. 	Which is the most suitable distillation column for separation of very closest boiling Point liquid mixture?

9. 	Write down the basic mass balance equation at steady state condition.

10. 	Write the basic rate equations.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the basic parameters required for location of plant layout?

12.	Differentiate between the orifice meter and venturimeter.

13. 	State the LMTD and give its Significance.

14. 	What is the significance of relative volatility?

15. 	What are the basic parameters required for design of batch reactor?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Draw a   plant layout  ethanol with neat schematic diagram .                				  (10)

b.	Draw symbols for various unit operations.                                             				    (5)

(OR)

17. 	Discuss about the Engineering materials selection for fabrication of process equipments.

18. 	Describe the designing steps for Venturimeter  and give its industrial  application.

(OR)

19. Explain   the general procedure for design of Gate valve with neat sketch.

20. 	Cold fluid is flowing through the heat exchanger at a rate of 15 m3/hr .It enters the heat exchanger at 303K and leaves at 328K.The hot thermic fluid enters the heat exchanger at the rate of 21m3/hr at a temperature of 388K .Find out the area of heat transfer required assuming the flow is counter current and overall heat transfer coefficient be 2490 W/m2 K.

Data : Density of cold fluid= 1000 kg /m3, density of thermic fluid = 950 kg/m3,Specific heat of cold fluid= 4.187 KJ/Kg.K, Specific heat of thermic fluid=2.93 KJ/Kg.K. Draw a neat block diagram. If Tubes is 150, estimate the length of H.E.

(OR)

[P.T.O]

21. 	Design a single effect evaporator to concentrate 10000kg/hr of  a chemical solution from10% to 	20% by weight . The feed enter at 30 ºC, Saturated  steam at 110 ºC  ( latent heat of 540 kcal/ 	kg) is available , Condensate leaves at saturation temperature,  The solution boils at 45 ºC 	(latent heat of 570 kcal/ kg), Specific heats  of all solution may be taken as 1.0. Overall heat 	transfer coefficient  may be taken as 1800 kcal/hr m2 ºC. Calculate i) Steam consumption  , 	kg/hr, ii) Heat transfer area . Given data , Enthalpy of feed, product and evaporator are 30 	kcal/kg, 45 kcal/ kg and  615kcal/kg .and latent  heat steam is 540 kcal/kg.

22. 	It is desired to fractionate a feed mixture of ethanol and propanol containing 65mol%ethanol.To a distillate containing 92mole% ethanol and a residue containing 93mole%propanol.The feed enters as a saturated vapour. a) determine the minimum reflux ratio b) for a reflux ratio of 4, determine the theoretical number of plates needed and    c)locate the feed plate point at its boiling point.

| X | 0 | 0.1 | 0.2 | 0.3 | 0.5 | 0.7 | 0.8 | 0.9 | 1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 0 | 0.19 | 0.34 | 0.47 | 0. 67 | 0.83 | 0.89 | 0.95 | 1.0 |

X,Y mole fraction of ethanol in liquid and vapour phase respectively.

(OR)

23. 	A counter current rotary dryer is to be designed for drying 20,000 kg/hr of wet salt containing 7.5% (wet basis) water to 0.25% (wet basis) water. The wet salt enters the dryer at 30 ºC and it has a specific heat of 0.25 kcal/kg ºC. Heated air at 150ºC with a wet bulb of 40ºC is available for drying the salt. If the outlet temperature of air and salt are 65ºC and 80ºC respectively, calculate the length and diameter of the dryer.

24. 	Write down  the designing procedure step by step  of an  ideal batch reactor.

(OR)

25. 	Derive the design equation for  non-ideal reactor.