UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12CH201 | Duration : | 3 hrs |
| Sub. Name : | APPLIED CHEMISTRY | Max. marks : | 100 |

Answer ALL questions

PART – A (10 X 1 = 10)

1.   Give the unit to express the amount of hardness in water.

2.   Give one example for chemical coagulant.

3.   The number of reactive sites in a monomer is called ______

4.   Name two resins used in making electrical wire?

5.   The calorific value of water gas is _________

6.   The efficiency of fuel is measured in terms of ___________.

7.   The oxidation potential of copper electrode is ___________.

8.    ______________ is the metal used as sacrificial anode.

9.    One example for solid lubricant.

10.  The desired quality of an abrasive is ----------.

PART – B (5 x 3 = 15 MARKS)

11. Calculate the temporary hardness and permanent hardness of a sample water containing: 	Mg(HCO3)2 = 14.6 mg/l; Ca(HCO3)2 = 16.2 mg/l; CaCl2=11.1mg/l;MgSO4=12.0mg/l

12.   Write a note on the preparation of poly vinyl chloride.

13.   Define octane number. Give two examples for anti-knocking agents.

14.   When zinc is partially immersed in aqueous NaCl solution, only the immersed portion undergoes                       	corrosion. Give reason.

15.    Briefly discuss about silicon carbide preparation and properties.

PART – C (5 X 15 = 75 MARKS)

16.  a. Describe Zeolite process of softening hard water with a neat diagram.  	               	  (8)

b. Write a note on boiler corrosion by dissolved oxygen. 				   	  (7)

(OR)

17.  a. Explain the steps involved in the removal of hardness by ion exchange method.         	(10)

b. What is break point chlorination? Explain its significance.				    	  (5)

18.  a.  Explain about compounding of plastics. 						  	(10)

b. Differentiate between thermoplastics and thermosetting plastics. 			    	  (5)

(OR)

19.  a.  How is bakelite prepared? Mention its uses. 					              	(12)

b. Write the preparation of any polymer by addition polymerization reaction.                  	  (3)

20. a. How is flue gas analysed by Orsat’s apparatus? 				            	(10)

b. Give the characteristics of a good fuel.						 	  	  (5)

(OR)

21. a. Describe ultimate analysis of coal and its significance. 					(10)

b. Mention the importance of proximate analysis. 					  	  (5)

22. a. Derive an expression for the emf of a concentration cell. 				  	  (6)

b. Explain the factors influencing corrosion with respect to the nature of the metal. 	  	  (9)

(OR)

23. a. Describe the construction and working of hydrogen – oxygen fuel cell. Give applications.	  (8)

b. Explain how corrosion can be controlled by cathodic protection.   				  (7)

24.  Explain in detail the properties of refractory material and classification with two examples.

(OR)

25. a. Briefly discuss about functions, and classification of lubricant.  				  (8)

b. Give the preparation and uses of boron carbide. 						  (7)