UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12EC204 | Duration : | 3 hrs |
| Sub. Name : | MEASUREMENTS AND INSTRUMENTATION | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | What is meant by measurement? | (1) |
| 2. | To increase Q factor of a coil, the wire should be a. long b. thick c. thin d. thin and thick | (1) |
| 3. | The most common method for measurement of low resistance is: (a) Wheatstone bridge (b) Potentiometer method (c) Voltmeter-ammeter method (d) Kelvin's double bridge | (1) |
| 4. | Induction type single phase energy meters measure electric energy in a.kW b.Wh c.kWh d.VAR | (1) |
| 5. | State the balance equation used in ac bridges. | (1) |
| 6. | The bridge circuit used to measure the capacitance is a. Wheatstone bridge b. Kelvin c. Schering Bridge d. Maxwell Bridge | (1) |
| 7. | Give one example of piezoelectric materials. | (1) |
| 8. | What is a transducer? | (1) |
| 9. | Which of the following is measured by using a vector voltmeter? (a) Amplifier gain and phase shift (b) Filler transfer functions (c)Complex insertion loss (d)All the above | (1) |
| 10. | The time base signal in a CRO is a: (a) Rectangular waveform (b) High frequency sinusoidal wave form (c) High frequency sawtooth wave form (d) Square wave form | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Mention the basic requirements of measurement. | (3) |
| 12 | What is meant by Caliberation? | (3) |
| 13 | Give the need of measurement in the bridge circuit. | (3) |
| 14 | Define piezo electric effect. | (3) |
| 15 | What is real time spectrum analyzer? | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |
| --- | --- | --- |
| 16. | Discuss the working principle of operation of Galvanometer type of instruments with its constructional diagram. Also derive the torque equation of the galvanometer. | (15) |
| (OR) |  |  |
| 17. | Describe in detail about the principle, construction and working of the moving iron instrument. | (15) |
| 18. | Explain the techniques available for adjustments to make the energy meter measure the energy consumed. | (15) |
| (OR) |  |  |
| 19. | Discuss the construction and operation of instrument transformers and calculate the power. | (15) |
| 20. | Describe how measurement of resistance can be done by using Wheatstone bridge. What types of resistance are suitable for measurement using this bridge? | (15) |
| (OR) |  |  |
| 21. | Explain the construction of Anderson bridge. Derive the unknown quantities at balance condition. | (15) |
| 22. | Explain about the various classifications of Transducers with an example of each type in detail. | (15) |
| (OR) |  |  |
| 23. | Explain the principle, characteristics and advantages of LVDT with a neat diagram. | (15) |
| 24. | With a neat block diagram explain how XY recorder can be measured using signal? Give the uses of XY recorder. | (15) |
| (OR) |  |  |
| 25. | Draw and explain the working principles and the block diagram of a Digital Voltmeter of ramp type. | (15) |

ALL THE BEST