UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12EC205 | Duration : | 3 hrs |
| Sub. Name : | DIGITAL ELECTRONICS | Max. marks : | 100 |

| Q. No. | Questions | Course outcome | Marks |
| --- | --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |  |
| 1. | Convert (41)10 to binary. | CO1 | (1) |
| 2. | Convert 0001 to Excess-3-code | CO1 | (1) |
| 3. | Draw the truth table of 2-input AND gate. | CO2 | (1) |
| 4. | With truth table draw the circuit diagram of half adder. | CO2 | (1) |
| 5. | Draw the logic diagram of NAND gate. | CO2 | (1) |
| 6. | Write the excitation table of J-K flip-flop. | CO2 | (1) |
| 7. | Define shift register. | CO2 | (1) |
| 8. | What do you mean by synchronous counter? | CO2 | (1) |
| 9. | Give the abbreviation of RTL. | CO2 | (1) |
| 10. | Define fan-out. | CO2 | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 11. | Find the complement of the function F1= x′yz′ + x′y′z | CO1 | (3) |
| 12. | Realize OR gate using NOR gate. | CO2 | (3) |
| 13. | With characteristic equation draw the circuit diagram of J-K flip-flop. | CO2 | (3) |
| 14. | Draw the state diagram of 2-bit binary counter. | CO2 | (3) |
| 15. | Define propagation delay. | CO2 | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 16. | a. | Convert (1010.011)2 to decimal. | CO1 | (2) |
|  | b. | Convert (0.6875)10 to binary. | CO1 | (2) |
|  | c. | Convert (0.513) 10 to Octal. | CO1 | (2) |
|  | d. | Convert (10110001101011.11110010)2 to hexadecimal. | CO1 | (2) |
|  | e. | Convert (2047)10 to hexadecimal. | CO1 | (2) |
|  | f. | Realize the following Boolean expression using basic gates. | CO2 | (5) |
| (OR) |  |  |  |  |
| 17. |  | Determine the prime implicants using Quine-Mc Cluskey method. F(w,x,y,z) = Σ (1,4,6,7,8,9,10,11,15) | CO1 | (15) |
| 18. | a. | With neat circuit diagram explain about full adder and realize a full adder with 2 half adders and an OR gate. | CO2 | (10) |
|  | b. | Implement the Boolean function with a Multiplexer. F(A,B,C) = Σ (1,3,5,6) | CO2 | (5) |
| (OR) |  |  |  |  |
| 19. | a. | Design BCD to decimal decoder. | CO2 | (10) |
|  | b. | Draw 2-4 line decoder with enable input. | CO2 | (5) |
| 20. | a. | With neat diagram explain the working of a master slave J-K flip-flop using NAND gate. | CO2 | (10) |
|  | b. | With characteristics table, draw the circuit diagram of T flip-flop. | CO2 | (5) |
| (OR) |  |  |  |  |
| 21. |  | Design a minimal clocked sequential circuit for the reduced state table shown below: Present State; Next State; Output. X=0; X=1; X=0; X=1. a; c; b; 0; 0. b; f; c; 0; 0. c; e; f; 1; 1. e; f; a; 0; 1. f; e; f; 1; 0. | CO2 | (15) |
| 22. |  | With truth table and circuit diagram explain the working of 4-bit up-down counter using J-K flip-flop. | CO2 | (15) |
| (OR) |  |  |  |  |
| 23. |  | Give the different type of shift registers and explain in detail about SISO shift register. | CO2 | (15) |
| 24. |  | Draw and explain the circuit of open collector output gate and realize the function Y= (AB + CD) ′ using open collector logic. | CO2 | (15) |
| (OR) |  |  |  |  |
| 25. |  | Explain about CMOS process and realize CMOS Inverter, CMOS 2-input NAND and NOR gate. | CO2 | (15) |

ALL THE BEST