Reg.No. _____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12EC232 | Duration : | 3 hrs |
| Sub. Name : | DIGITAL DESIGN USING VHDL | Max. marks : | 100 |

| Q. No. | Questions PART-A(10X1=10 MARKS) | Marks |
| --- | --- | --- |
| 1. | Why PLDs are used? | (1) |
| 2. | In PLA AND arrays are ______________OR arrays are___________. | (1) |
| 3. | Define RTL. | (1) |
| 4. | Name the different programming technologies. | (1) |
| 5. | Write about emulation. | (1) |
| 6. | Define CPLD. | (1) |
| 7. | Define full custom design. | (1) |
| 8. | Give the syntax of CASE statement. | (1) |
| 9. | Define entity declaration. | (1) |
| 10. | ______________ delay is called default delay. | (1) |

PART B (5 X 3= 15 MARKS)

| 11 | Design the function using ROM F1 (A1, A0)=Σm (0, 1) F2 (A1, A0)= Σm (1,2, 3) | (3) |
| --- | --- | --- |
| 12 | With neat schematic explain the RTL notation Y A v B (A & B are 3 bits). | (3) |
| 13 | Discuss about standard cell design. | (3) |
| 14 | With examples describe in detail about WAIT statements. | (3) |
| 15 | By using CASE statement write VHDL code for a 4X1 multiplexer. | (3) |

PART C (5 X 15= 75 MARKS)

| 16. |  | Implement the functions using PLA A(x,y,z)= Σm(0,1,4,5) B(x,y,z)= Σm(0,2,4,6) C (x,y,z) = Σm(2,6) | (15) |
| --- | --- | --- | --- |
|  |  | (OR) |  |
| 17. | a. | Implement the following Boolean logic equations in PLA Z=Q2X+Q3X D1=XQ2Q3 D2=Q2Q3+XQ1+XQ2Q3 D3=Q2X+Q3X | (5) |
|  | b. | Implement the binary to gray code converter (4 bits) using PROM where A, B, C, D are inputs and W, X, Y, Z are outputs. | (10) |
| 18. | a. | Write a RTL description for the following logic system and implement it. “The data on the X lines (2 in number) are to be loaded in a Register A. The data in A are then to be complemented in Register C. With the data in C, next shifted into Register B, the data in Register A and Register B is then to be ORed, with the result being stored in Register C again. Finally the data in register C are to be placed on Z output lines. All the registers are 2-bit long”. | (10) |
|  | b. | Discuss about ROM Simulation. | (5) |
|  |  | (OR) |  |
| 19. | a. | With the design flow, lists the different operators and sub-programs used in ABEL. | (10) |
|  | b. | Describe the organization of a hypothetical tiny computer with a neat diagram. | (5) |
| 20. |  | Explain in detail about different types of Gate–array based design with neat diagrams. | (15) |
|  |  | (OR) |  |
| 21. |  | With neat diagram explain in detail about Xilinx 3000 FPGA and also describe how a 32-bit LUT can realize different functions. | (15) |
| 22. | a. | Write a VHDL Program for four bit full adder, using the structural description .Draw its basic structure showing the inputs and outputs signals. | (10) |
|  | b. | List the types of data operators available in VHDL with examples. | (5) |
|  |  | (OR) |  |
| 23. | a. | Explain in detail about different scalar data types. | (10) |
|  | b. | Write a VHDL program for half adder using dataflow modeling. | (5) |
| 24. | a. | Design a 2 bit magnitude comparator in Behavioral modeling using VHDL (use if-then-else OR CASE statement). | (10) |
|  | b. | Write the syntax of component declaration statement in structural modeling. | (5) |
|  |  | (OR) |  |
| 25. |  | With neat block diagram discuss about test bench and write the test bench for half adder. | (15) |

ALL THE BEST