Reg.No. _____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

Subject Code: 12EE257/EE271	    			                    					       Time: 3 hours

Subject Title: HIGH VOLTAGE ENGINEERING 		                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

- The objective of good line design is to _______the number of outages caused by lightning.

- How does switching over voltage originate?

- Paper insulation is mainly used in ____________.

- What is parallel plate capacitor?

- Distinguish between electromagnetic and electrostatic machines.

- Define impulse voltage.

- Sphere gap is used for measurement of_______________.

- Give any two advantage of electrostatic voltmeter.

- What is bending test in cables?

- Reproduce any two tests for testing cables.

PART – B (5 x 3 = 15 MARKS)

11. Reproduce the time response of RC series circuit.

12. Give the difference between photo ionization and photo electric emission.

13. Write a brief note on resonant transformer method.

14. Express the output voltage of voltage diverter.

15. Mention various tests done on CB.

PART – C (5 x 15 = 75 MARKS)

16. a) Elaborate the insulation coordination among all substation equipments in voltage	level. 	 (10)

b) A surge of 15 kV magnitude travels along a cable towards its junction with an overhead line. The   inductance and capacitance of the cable and overhead line are 0.3 mH, 0.4 μF respectively and 1.5 mH, 0.012 μF respectively per km. Find the voltage rise at the junction due to the surge.		  (5)

(OR)

17. 	a) Define ground wire.  Discuss its location with respect to power conductors.				 (10)

b) Express the transient current waveform in RC circuit	.							  (5)

18. Describe various factors that influence breakdown in a gas.

(OR)

19. Explain the different mechanisms by which breakdown occurs in solid dielectrics in practice.

20. Explain clearly the basic principle of operation of an electrostatic generator. Describe with neat diagram the principle of operation, application and limitations of Van de Graf generator.

(OR)

21. Why is a Cockcroft-Walton circuit preferred for voltage multiplier circuits? Explain its working with a schematic diagram.

22. Draw a neat schematic diagram of a generator voltmeter and explain its operation and give the application.

(OR)

23. What is Rogowski Coil? Explain with a neat diagram its principle of operation for measurement of high impulse currents.

24. List out various tests to be carried out on a cable and give a brief account of each test.

(OR)

25. Explain briefly various tests to be carried out on a power transformer.