Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec 2016

Subject Title :	THERMODYNAMICS								      	     Time : 3 hours

Subject Code:	12FP202						 				       Maximum Marks: 100

#### Answer ALL questions

(Use of approved Thermodynamics property tables, Steam tables are permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	State Zeroth law of thermodynamics.

2.	Define closed and open system with an example.

3.	Define quasi-static process.

4.	Differentiate between heat and work with an example.

5.	A refrigerator and heat pump operate between the same temperature limits. If COP of the refrigerator is 4, the COP of the heat pump would be?

6.	Write the air standard efficiency of Otto cycle?

7.	Define entropy?

8.	What do you understand by principle of increase of entropy

9.	What is a triple point of a substance?

10.	Define an ideal gas?

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short on thermodynamic equilibrium.

12.	Differentiate between a point function from a path function with any example.

13.	A simple saturated vapour compression refrigeration cycle has the following state points

(i) enthalpy after compression = 420 kJ/kg. Enthalpy after throttling =120 kJ/kg, enthalpy before compression = 370 kJ/kg. Determine the COP of the refrigerator.

14.	What are conditions for reversibility?

15.	Draw the phase equilibrium diagram for a pure substance on T-s plot with relevant constant property lines.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the temperature which has the same value on both the Celsius and Fahrenheit scale.

(OR)

17.	A piston cylinder device operates 1 kg of fluid at 20 atm. pressure. The initial volume is

0.04 m3. The fluid is allowed to expand reversibly following a process pV1.45=constant, so that the volume becomes double. The fluid is then cooled at constant pressure until the piston comes back to the original position. Keeping the piston unaltered, heat is added reversibly to restore it to the initial pressure. Calculate the work done in the cycle.

18.	A stationary mass of gas is compressed without friction from an initial state of 0.3m3 and

0.105 MPa to a final state of 0.15 m3 and 0.105 MPa, the pressure remaining constant during the process. There is a transfer of 37.6 kJ of heat from the gas during the process. How much does the internal energy of the gas change?

(OR)

19.	In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure, and velocity at the inlet are 0.37 m3/kg, 600 kPa, 16 m/s. The inlet is 32 m above the floor, and the discharge pipe is at floor level. The discharge conditions are              0.62 m3/kg, 100 kPa and 270 m/s. The total heat loss between the inlet and discharge is 9 kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease, and how much?

20.	A cyclic heat engine operates between a source temperature of 800 oC and a sink temperature of 30 oC. What is the least rate of heat rejection per kW net output of the engine?

(OR)

21.	A Carnot engine absorbs 200 J of heat from a reservoir at the temperature of the normal boiling point of water and rejects heat to a reservoir at the temperature of the triple point of water. Find the heat rejected, work done by the engine and the thermal efficiency.

22.	A fluid undergoes a reversible adiabatic compression from 0.5 MPa, 0.2 m3 to 0.05 m3 according to the law, pV1.3=constant. Determine the change in enthalpy, internal energy and entropy, and the heat transfer and work transfer during the process.

(OR)

23.	At standard temperature and pressure, 8.4 litres of oxygen and 14 litres of hydrogen mix with each other completely in an insulated chamber. Calculate the entropy change for the process assuming both gases behave as ideal gases.

24.	Derive Maxwell’s equations.

(OR)

25.	A vessel of volume 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 250 oC. The mass of the liquid present is 9 kg. Find the pressure, the mass, the specific volume, the enthalpy, the entropy, and the internal energy.