UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12ME234 | Duration : | 3 hrs |
| Sub. Name : | FINITE ELEMENT ANALYSIS | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define the term ‘finite element’ | (1) |
| 2. | How do you calculate the size of element stiffness matrix? | (1) |
| 3. | Any member subjected to axial load is called --------------- | (1) |
| 4. | Write the different type of nodes | (1) |
| 5. | Define global co-ordinate system | (1) |
| 6. | Mention the degrees of freedom in c1 continuity | (1) |
| 7. | Write the interpolation polynomial equation for eight noded quadrilateral element | (1) |
| 8. | Force that acts throughout the volume of a body is called ----------- | (1) |
| 9. | Define incompressible fluid | (1) |
| 10. | Write the governing differential equation applicable for any field problem | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Write the general applicability of finite element method to fluid mechanics problem. | (3) |
| 12 | List the properties of shape function. | (3) |
| 13 | Define area co-ordinate system. | (3) |
| 14 | Write the purpose of higher order elements. List its advantages. | (3) |
| 15 | Write the stiffness matrix for a steady state heat transfer in 1D fin subjected to convective heat loss on both sides. | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | a. | Consider the differential equation for the problem as, d2ydx2+400x2=0; 0 £x £ 1 with the boundary conditions y (0) = 0 and y(1) = 0. Assume suitable trial function and find the solution of the problem using two term trial function and using point collocation and sub domain methods. | 15 |
| (OR) |  |  |  |
| 17. | a. | Solve the differential equation for a physical problem expressed as d2ydx2+100=0; 0 £x £ 10 with the boundary conditions y (0) = 0 and y(10) = 0 using point collocation method, Galerkin’s method, least square method and sub domain method. The trial function used is y=ax(x-10). | 15 |
| 18. | a. | Calculate the deflection and slope for the beam fixed at one end and supported by a roller at other end (shown in Figure1). Assume E = 2x107 N/cm2 , I = 2500cm4 , L=1000cm and P = 20 kN Figure1 | 15 |
| (OR) |  |  |  |
| 19. | a. | Derive the stiffness matrix for 1D truss element | 10 |
|  | b. | List the properties of stiffness matrix | 5 |
| 20. | a. | Determine three points on the 500C contour line for the rectangular element shown in Figure2. The nodal values are T1=420C, T2= 540C,T3= 560C and T4= 460C. Nodal coordinates are node1 (5,3), node2 (8,3), node 3(8,5) and node4( 5,5). S x Y 2(8,3) 1(5,3) 3 (8,5) 4(5,5) t Figure2 | 15 |
| (OR) |  |  |  |
| 21. | a. | Figure 3 i (0,0)  j(4,0.5) k(2,5) A X Y Evaluate the element shape functions and calculate the value of pressure at point A shown in Figure3.The nodal values of displacement are Φi = 40N/cm2 ;Φj = 34N/cm2;Φk= 46N/cm2. Point A is located at (2, 1.5). (All Dimensions are in cm) | 15 |
| 22. | a. | Explain lagrangian element and derive its shape function | 15 |
| (OR) |  |  |  |
| 23. | a. | For the stepped bar loaded as shown in Figure4, determine nodal displacements and element stresses. E = 20 x 106 N/cm2; Rise in temperature is 10C ;. = 11 x 10 -6 perC A=24cm2 X A=15cm2 75 cm 75 cm 60 cm 10KN 20KN Figure4 | 15 |
| 24. | a. | Determine the temperature distribution in a one dimensional circular cross section fin shown in Figure 5. The fin is of 8cm long and 1cm diameter. Assume that convection heat loss occurs from the right end of the fin.(use 2 element idealization) L=8 cm 100C 1 cm diameter φf=20∘C, h=0.2Wattscm2 ∘C , k=2Wattscm ∘C Figure5 | 15 |
| (OR) |  |  |  |
| 25. | a. | A wall of an industrial oven consists of three different materials. The first layer is composed of 5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal conductivity of 0.72 W/m K. The inside (inlet) wall temperature of the oven is 200°C, and the outside air is 30°C with a convection coefficient of 40W/m2K (Convective heat loss occurs at right side). Determine the temperature distribution along the composite wall. Consider area of the element as 1m2. | 15 |

ALL THE BEST