UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 12ME308 | Duration : | 3hrs |
| Sub. Name : | FINITE ELEMENT ANALYSIS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Marks |
| --- | --- | --- | --- |
| 1. | a. | Explain the step by step procedure of FEA | 10 |
|  | b. | Discuss the factors to be considered while discretization. | 10 |
| (OR) |  |  |  |
| 2. | a. | Solve the following system of equation using Choleski method: 3X1 +X2 +X3 =2 X1 +4X2 +X3 =12 2X1 +X2 +2X3 =10 | 20 |
| 3. | a. | Consider the differential equation for the problem as, d2ydx2+300x2=0 ; 0 £x £ 1 with the boundary conditions y (0) = 0 and y(1) = 0. Find the solution of the problem using a one coefficient trial function as Y=AX(1-X3). Use (i) Point collocation method (ii) sub domain method (iii) Least square method (iv) Galerkin’s method. | 20 |
| (OR) |  |  |  |
| 4. | a. | For the triangular element shown in Figure1, the nodal values of displacement are: u1 = 0.001; u2 = 0.003; u3 = -0.002 v1 = -0.004; v2 = 0.002; v3 = 0.005 Find the displacement u,v at point P (2, 5) within the element. (All Dimensions are in mm) u  (1,1) 1 2 (3,4) 3 (2, 7) v pu Figure 1 | 20 |
| 5. | a. | Find the stresses induced in the axially loaded stepped bar shown in Figure 2. The bar has cross sectional areas A1 and A2 over the lengths L1 and L2 respectively. Assume the following datas: A1= 2cm2 and A2= 1cm2, L1=L2=10cm, E1=E2=2x107N/cm2, P=100N. 2 10 cm 10 cm 1 3 1 2 P Figure2. | 20 |
| (OR) |  |  |  |
| 6. | a. | Find the natural frequencies of longitudinal vibrations of the unconstrained stepped bar shown in Figure 3. Figure 3 length (1)=L/2 length (2)=L/2 Q3 Q1 Q2 X Element 1 A (1) =2A Element 2 A (2) = A 1 2 | 20 |
| 7. | a. | For the smooth pipe of variable cross section shown in Figure 4, determine the potential at the junctions, velocities in each section of the pipe and the volumetric flow rate. The potential at the left end is ɸ1=10m and that at the right end is ɸ4=2m. Permeability coefficient k=1m/s. 1 2 3 4 A1=3m2 A2=2m2 A3=1m2 1 m 1 m 1 m Figure 4 | 20 |
| (OR) |  |  |  |
| 8. | a. | Determine the temperature distribution in a one dimensional fin which is rectangular in shape, 8cm long, 4cm wide and 1cm thick. The left side of the fin (inlet) is kept at 80oC. Assume that convection heat loss occurs from the right end of the fin. (use 2 element idealization) φf=20∘C, h=0.1Wattscm2 ∘C ; k=3Wattscm ∘C | 20 |
|  |  | Compulsory: |  |
| 9. | a. |  1cm y x A M B 1cm Determine element shear stresses for a steel shaft of square cross section shown in Figure5 Take G=8x106 N/cm2, Angle of twist=0.01o/cm. Figure.5 | 20 |

ALL THE BEST