Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Title :	BIOPROCESS CALCULATIONS					      	     Time : 3 hours

Subject Code:	12BT219						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	1 cm2 = ______________ m2.

2.	Molecular weight of H2SO4 = ______________.

a.	98                  b.	49                       c.	40                        d.	54.75

3.	______________ process, liquid mixture is converted into distillate and bottom product.

a.	Distillation    	b.	Evaporation       	c.	Mixing          	d.	Filtration

4.	Overall material balance for evaporation process is ______________.

5.	Yield =  ______________.

6.	% conversion = ______________.

7.	Kinetic energy is defined as ______________.

8.	Heat = ______________.

9.	Work =______________.

10.	Calorie is defined as ______________.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert a volumetric flow rate of 2 m3/s to l/s.

12.	What is the average molecular weight of air and its composition by weight?

13.	Define Stoichiometric equation.

14.	Short notes on Hess’s law of constant heat summation.

15.	Define Heat of formation.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain 	a.	Ideal gas law    b.	Dalton’s law 		c. 	Amagat’s law

(OR)

17.	Explain the concentration of a solution

a.	Normality            b.	Molality                    	c.	Molarity

18.	Draw the symbols for the following unit operations

a.	Evaporator          	b.	Distillation column  	c.	Dryer

(OR)

19.	A single effect evaporator is fed with 10,000 kg/h of weak liquor containing 15% caustic by weight and is concentrated to get thick liquor containing 40% by weight caustic. Calculate

a) kg/h of water evaporated and 	b) kg/h of thick liquor obtained.

20.	In production of Sulphur trioxide, 100 kmol of SO2 and 100 kmol of O2 are fed to a reactor. If the percent conversion of SO2 is 80, calculate the composition of the product steam on mole basis.

[P.T.O]

(OR)

21.	Ethylene oxide is produced by oxidation of ethylene. 100 kmol of ethylene are fed to a reactor and the product is found to contain 80 kmol ethylene oxide and 10 kmol CO2. Calculate

a) the percent conversion of ethylene and b) the percent yield of ethylene oxide

22.	Methane gas is heated from 303 K to 523K at atmospheric pressure. Calculate the heat added per kmol methane,using the heat capacity data given below:

Cp = a+bT+cT2+dT3, kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| Methane | 19.2494 | 52.1135 | 11.973 | -11.3173 |

(OR)

23.	Toluene is to be heated from 290K to 350K at the rate of 250gm/sec. Calculate the heat to be supplied to toluene using the heat capacity data given below

Cp = a+bT+cT2+dT3, kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| Toluene | 1.8083 | 812.223 | -151.67 | 1630.01 |

24.	When liquid benzene is completely burned to carbon dioxide and liquid water, the standard heat of combustion is – 3267.6 kJ/mol. The standard heat of combustion of hydrogen to liquid water is – 285.83 kJ/mol and that of carbon to carbon dioxide gas is – 393.51 kJ/mol. Calculate the standard heat of formation of liquid benzene.

(OR)

25.	Calculate the heat of formation of phenol crystals at 298.15 K from its elements using the following data: standard heat of formation of carbon dioxide is -393.51 kJ/mol, standard heat of formation of water is -285.83 kJ/mol and heat of combustion of phenol crystals at 298.15 K is

-3053.15 kJ/mol.