Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec 2016

Subject Title :	CONSTRUCTION PRACTICES						      	     Time : 3 hours

Subject Code:	12CE238						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the types of brick blocks used in recent construction.

2.	Classify the iron metal.

3.	What do you mean by bye - laws?

4.	In which foundation a cantilever beam is provided to join two footings.

5.	What do you mean by bond in masonry?

6.	What are the materials used in RCC work?

7.	Name the materials used for formworks.

8.	Define termite proofing.

9.	Which equipment is used for handling structural elements?

10.	What do you mean by dredging?

PART – B  (5 x 3 = 15 MARKS)

11.	‘Durability of timber depends on the seasoning methods’. Describe any two seasoning methods of timber.

12.	Brief the foundation requirements.

13.	Being a quality check Engineer, how will you test the characteristics of cement in the field?

14.	Brief the methods of concrete curing in the field.

15.	Brief the safety measures to be taken in the construction field.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Classify the building materials.													    (6)

b. 	Differentiate brick masonry and stone masonry.									    (9)

(OR)

17.	a.	For various construction different types of aggregate are used, Classify aggregates.	    (7)

b.  	Discuss Geosynthetics and its application in the construction field.					    (8)

18.	Summarize the principles and the factors to be considered for planning a civil engineering structure.

(OR)

19.	a.	Recommend the types of foundation in black cotton soil and explain with neat sketch.  (8)

b.	“Failure in foundation leads to global failure of building”. Discuss with respect to remedial 	measures.							            									    (7)

20.	Discuss the various types of cement and emphasis on its application in the construction field.

(OR)

21.	Outline the process of manufacture of concrete and explain the test adopted to check its quality both in fresh and hardened state.

[P.T.O]

22.	Elaborate the construction sequence of RCC framed structures. Explain how it differs from the load bearing structure.

(OR)

23.	a. 	Compare the construction methods adopted in cast-in-situ and precast concrete structure. 																						  (10)

b. 	Discuss the underpinning methods with neat sketches.  							    (5)

24.	Discuss the equipments used during the concrete construction at various stages of concreting.

(OR)

25.	Briefly explain the selection, classification and uses of dewatering and pumping equipment.