# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

# Subject Title: 	INDUSTRIAL POWER SYSTEM ANALYSIS AND DESIGN												       									Time : 3 hours

# Subject Code: 	14EE3023										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What are the different types of faults in the power system? Explain the role of neural			conductor in the power system.												     (10)

b.	What are the various sources of harmonics in the power system and describe the 				maximum harmonic amplitude of each source.								     (10)

2.	a.	Explain what happens if capacitor banks are added to improve the power factor above 	unity with illustrations.													      (10)

b.	The power factor of a 300 kW induction motor load is 0.86. It is desired to improve the power factor to 0.94. What is the required KVAR of the shunt capacitor bank? What types of technical issues are expected in applying power factor correction to an induction motor?																	      (10)

(OR)

3.	a.	Explain the circuit for the analysis of Voltage magnification when the capacitor banks 	are connected at high voltage and low voltage power systems.					      (10)

b.	Describe the concept of back-to-back switching with two capacitor banks without series reactor and with series reactor in the switching circuit with necessary diagrams.	      (10)

4.	a.	Explain the acceptable criteria for harmonics according to IEEE 519 standards in power 	system.																	      (10)

b.	The simple notch filter is effective at only one frequency. What dictates the filtering 	frequency? Are there any mechanisms to alter the filtering frequency?			      (10)

(OR)

5.	a.	Briefly describe the power system response to harmonics with necessary illustrations. 																			      (10)

b.	Explain the procedure for harmonic evaluation in the power network.			      (10)

6.	a.	Describe the effect of flickering in electrical traction and particle accelerator.	      (10)

b.	What is the difference between AC arc furnace and DC arc furnace? What are the factors 	used in the selection of the arc furnace?										      (10)

(OR)

7.	a.	Briefly explain the flicker criteria in electrical loads with schematic diagrams.	      (10)

b.	How do you minimize the flicker effects in the electrical supply?				      (10)

8.	a.	Explain the concept of touch voltage in the grid using equivalent circuit and determine 	the procedure to compute the coefficient factor.								      (10)

b.	Explain the procedure to compute the ground grid calculations with illustrations.	      (10)

(OR)

9.	Briefly describe the methods to improve the performance of grounding grids with necessary illustrations.