# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  Nov/Dec 2016

# Subject Title: 	ADVANCED TURBO MACHINERY					       Time : 3 hours

# Subject Code: 	15ME3011								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Compare a radial flow compressor with an axial flow compressor.         			      (10)

b.	What is surging in axial flow compressors? What are its effects?           			      (10)

2.	The conditions of air at the entry of an axial compressor stage are P1 = 768 mm Hg and

T1 = 314 K. the air angles are

β1  = 51o , β9 = 9o, α1=   α 3 = 7o

the mean diameter and peripheral speed are 50 cm and 100 m/s respectively. Mass-flow rate through the stage is 25 kg/s: the work done factor is 0.95 and mechanical efficiency of 88% determine

a.   air angle at the stator entry

b.   blade height at entry and the hub –tip diameter ratio,

c.   stage loading coefficient,

d.   stage pressure ratio and

e.   the power required to drive the stage.

(OR)

3. 	Derive Euler’s turbine and compressor equations. Hence show that for axial machines, Euler’s work is given byα

± u dcθ

± m (u1cθ1 – u2cθ2)

±12c12−c22+w12−w22

4.	a.	What is infinitesimal stage efficiency in the expansion through a turbine? How it is 	determined?																      (10)

b.	Show that the index n of polytrophic expansion in a turbine of infinitesimal stage 	efficiency ηpis

n=γγ−γ−1ηp.															       (10)

(OR)

5.	a.	Derive L = ρcmГ for a cascade of infinite pitch, where the blade behaves as an isolated 	blade.																	      (10)

b.	State and brief the Aerodynamic loses occurring in a turbine blade cascade.		      (10)

6.	a.	An axial follow compressor stage has the following data:

Temperature and pressure at entry        300K,  1.0 bar

Degree of reaction                                 50%

Mean blade ring diameter                      36 cm

Rotational speed                                     1800 rpm

Blade height at entry                              6 cm

[P.T.O]

Air angles at rotor and stator exit            25o

Axial velocity                                         180m/s

Work-done factor                                   0.88

Stage efficiency                                      85%

Mechanical efficiency                             96.7%

Determine: (a) Air angles at the rotor and stator entry, (b) the mass-flow rate of air,

(c) the power required to drive the compressor,(d) the loading coefficient, (e) the pressure ratio developed by the stage and (f) the Mach number at the rotor entry.

(OR)

7.	a.	Draw the T-h curves for the hot gases and steam in the H.P and L.P. circuits. Indicate the 	pinch points.																      (10)

b.	Write down ten advantages and five disadvantages of combined gas and steam turbine 	power plants.                                                                 						      (10)

(OR)

8.	An inward flow radial turbine has the following data.

Power                                                                           150 kW

Speed                                                                          32000 rpm

Outer diameter of the impeller                                   20 cm

Inner diameter of the impeller                                     8 cm

Absolute velocity of gas at entry                                 387 m/s

Absolute velocity of gas at exit                                    193m/s

The gas enters the impeller raidally. Construct the velocity triangles at the entry and exit of the impeller and determine:

The mass-flow rate and

The percentage energy transferto the change of radius.

9.	a.	Draw inlet and outlet velocity triangles for a Pelton turbine and explain them briefly.

(10)

b.	A model of the Pelton turbine runs at 102 rpm under a head of 30 m and                        	discharge 0.345 m3/s. Calculate the power, flow rate and speed of a prototype working 	under a head of 1500m. What is its specific speed?                   (10)