# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  Nov /Dec 2016

# Subject Title: 	RADIATION TREATMENT AND PLANNING		       Time : 3 hours

# Subject Code: 	15PH3021								             	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. 	Compulsory :

a.	PDD for a 15 cm x 15 cm field size, 10 cm depth, and 80 cm SSD is 58.4 for Co-60 	beam. Find PDD for the same field size and depth for a 100 cm SSD. 				(2)

b.	A patient is to be treated with an orthovoltage beam having a HVL of 3 mm Cu. Supposing that the machine is calibrated in terms of exposure rate in air, find the time required to deliver 200 cGy at 5 cm depth, given the following data: exposure rate = 100 R/min at 50 cm, field size = 8 cm x 8 cm, SSD = 50 cm, PDD= 64.8, BSF= 1.20, and cGy/R= 0.95. 																(5)

c.	Draw the central axis depth dose distribution map of different quality beams. 		(3)     	d.	Explain the Clarkson’s method for dose calculation in irregular fields. 			      (10)

2. 	a.	Draw the configuration of isocentric medical linac. 								(5)

b.	Explain the construction and working of a cyclotron. 								(5)

c.	Explain the typical therapy X-ray tube with neat diagram. 						     (10)

(OR)

3.	a.	Tabulate any 10 differences between telecobalt and linac units.   				      (10)

b.	Write a short note on production and transport of electron beam in linac. 			(5)

c.	Discuss the various dose monitoring systems in linac. 							(5)

4.	a.	Define the target and critical structure volumes given by ICRU 50 and 62. 		     (15)

b.	Compare the conventional simulator with computed tomography simulator. 			(5)

(OR)

5.	a.	Explain the correction methods for contour irregularities and tissue inhomogeneities.

(10)

b.	Explain how DVH are used to evaluate any treatment plan. 					      (10)

6.	a.	Discuss the two types of electron energy losses. 									(5)

b.	What are the detectors determine the electron depth dose and isodose distribution? 	Explain it. 																      (10)

c.	Discuss the interaction of electrons with an absorbing medium. 					(5)

(OR)

7.	a.	Explain in detail about the various dosimetric parameters of an electron beam. 	      (15)

b.	Write a note on “Edge effect” of an electron beam.								(5)

8. 	a.	Write an essay on stereotactic irradiation. 									      (10)

b.	Describe about total body irradiation. 										      (10)

(OR)

9.	a.	Write about intraoperative radiotherapy. 									      (10)

b.	Discuss the various techniques behind the image guided radiotherapy. 			      (10)