# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

# Subject Title: 	MATHEMATICAL THEORY OF ELASTICITY		       Time : 3 hours

# Subject Code: 	16MA3001										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- Determine the strain and rotation tensors eij and ωij for the following displacement field:

u=Axy, v=Bxz2, w=C(x2+y2) where A, B and C are constants.				      (10)

b.	For isotropic materials shows that the principal axes of strain coincide with the principal 	axes of stress.  Further, show that the principal stresses can be expressed in terms of the 	principal strains as σi=2μei+λekk.										      (10)

2.	a.	Derive the Winkler-Bach formula for bending of curved beams.				      (10)

b.	For the torsion of an elliptical section, show that the resultant shear stress at any point within the cross-section is tangent to an ellipse that passes through the point and has the same ratio of major  to minor axes as that of the boundary ellipse.				      (10)

(OR)

3. 	Derive the expression for maximum shear stress and displacement for a circular and elliptical bar subjected to torsion.

4. 	a.	A steel shaft of 10 cm diameter is shrunk inside a bronze cylinder of 25 cm outer 	diameter.  The shrunk allowance is 1 part per 1000 (i.e., 0.005 cm difference between the 	radii).  Find the tangential stress in the bronze cylinder at the inside and outer radii and 	the stress in the shaft.  Adopt Esteel = 214 x 106 kPa, Ebronze = 107 x 106 kPa and υ = 0.3 	for both metals.															      (10)

b.	Derive the expression for stresses due to gravitation.							      (10)

(OR)

5.	a.	A flat steel disk of 75 cm outside diameter with a 15 cm diameter hole is shrunk around a 	solid steel shaft.  The shrink-fit allowance is 1 part in 1000 (i.e., an allowance of 0.0075 	cm in radius).  E = 214 x 106 kPa.

(i) What are the stresses due to shrink-fit?

(ii) At what rpm will the shrink-fit lossen up as a result of rotation?

(iii) What is the circumferential stress in the disk when spinning at the above speed?

Assume that the same equations as for the disk are applicable to the solid rotating shaft also.																	      (15)

b.	A solid steel propeller shaft, 60 cm in diameter, is rotating at a speed of 300 rpm.  If the shaft is constrained at its ends so that it cannot expand or contract longitudinally, calculate the total longitudinal thrust over a cross section due to rotational stresses.  Poisson’s ratio = 0.3.  The weight of steel may be taken as 0.077938 N/cm2.			(5)

6.	a.	Show that the resultant circumferential force across any radial section of a hollow disk 	subjected to thermal loading is zero.										      (10)

b.	Determine the bending moment due to the circumferential stress across a section of a 	thin hollow disk subjected to radial thermal variation.							      (10)

(OR)

[P.T.O]

7.	If an isotropic solid is heated non uniformly to a temperature distribution T(x, y, z) and the material has unrestricted thermal expansion, the resulting strain will be eij = αTδij.  Show that this case can only occur if the temperature is a linear function of the coordinates; that is,

T = ax + by +cz +d.

8.	Obtain the expression for various elastic constants for the following cases:

a.	Transversely isotropic														      (10)

b.	An epoxy matrix is reinforced with 40% by volume E-glass fibres to produce a 20 mm diameter composite to carry a load of 25 kN.  Calculate the stress acting on the fibre elements.  The modulus of epoxy is 3 GPa and that of glass fibre is 72.4 GPa.	      (10)

(OR)

9.	a.	Consider a graphite-epoxy laminate whose elastic constants along and perpendicular to 	the fibres are as follows.  Exx = 181 GPa, Eyy = 10.3 GPa, Gxy  = 7.17 GPa, υyx = 0.28 and 	υxy = 0.01594.  Obtain the compliance coefficients appropriate to x’y’ axes which are at  	(a) +30 degree counter clockwise to xy axes and (b) +90 degree to xy axes.	 	      (10)

b.	A unidirectionally reinforced composite of ‘Toray’ filament and ‘Nameo’ resin  has the 	following modulii and Poisson’s ratio.  Exx = 181 GPa, Eyy = 10.3 GPa, Gxy  = 7.17 GPa  	and υxy = 0.0159.  (1-VxyVyx)-1 = 1.0045.  Estimate the components of modulii for an off-	axis orientation of  (a) Ɵ = +30 degree and (b) Ɵ = +45 degree.					      (10)