Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	EC245											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is zero padding?

2.	What is the relation between DTFT and DFT.

3.	If x(n) = x(–n) then the signal is called _________.

4.	What are the advantages of FIR filter?

5.	What is the relation between digital and analog frequency in bilinear transformation?

6.	Sketch the magnitude response of Chebyshev type –1 filters.

7.	What are the two types of limit cycles?

8.	What is the range of error in rounding?

9.	What are the special features of digital signal processors?

10.	The _________ architecture employ separate buses for program and data.

PART – B  (5 x 3 = 15 MARKS)

11.	List the differences between linear convolution and circular convolution.

12.	What are the conditions to be satisfied for constant phase delay in linear phase FIR filters?

13.	Sketch the mapping of s-plane to z-plane in bilinear transformation.

14.	Compare the fixed point and floating point number representations.

15.	What are the addressing modes of TMS320C5x processing?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Find the Linear and circular convolution of the sequences x(n) and h(n)

xn=1, 0.5↑ and hn=0.5, 1↑.											            (7)

b. 	Compute the DFTs of the sequence xn=cosxn=nπ2.    						    (8)

(OR)

17.	Compute eight point DFT of a sequence x (n) = {1, 2, 1, 2, 1, 3, 1, 3} using DIT algorithm.

18.	a.	What is meant by Optimum equiripple design criterion? Why it is followed?       		    (4)

b.	Write the procedure for FIR filter design by frequency sampling method.            		    (4)

c.	Obtain the cascade realization of system function H z=1+2z−1−z−21+z−1−z−2                      																			    (7)

(OR)

19. 	Design a linear phase FIR band pass filter to pass frequencies in the range ωc= 0.4Π to 0.65 Π rad/sample using Hanning window taking N=7.

[P.T.O]

20.	Design a digital Butterworth filter with the following specification

0.8≤Hejω≤1, 0≤ω≤0.2π

Hejω≤0.4, 0.32π≤ω≤π

with T=1 sec. Apply impulse invariant transformation.

(OR)

21. 	a.	Explain the design procedure involved in designing a low pass digital Chebyshev filter.

(5)

b.	For the analog transfer function,  Has=2s+1s+2,	, determine H (z) if (a) T=1 sec and(b)T=0.1sec and (b) T=0.1sec using impulse invariant transformation.		     	  (10)

22.	An LTI system is characterized by the difference equation, y(n)=0.68y(n–1)+0.15x(n). The input signal x(n) has a range of -5V to +5V, represented by 8-bits. Find the quantization step size, variance of the error signal and variance of the quantization noise at the output.

(OR)

23.	a. 	Explain finite word length effects in realization of digital filters.                                  	  (10)

b. 	Write short notes:   (a) Truncation (b) Rounding.	                                                             (5)

24.	Discuss about adaptive noise cancellation using LMS algorithm in detail.

(OR)

25. 	Explain about extended parallelism techniques commonly followed in DSP processors.