Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14AE2021 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | GAS DYNAMICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write short notes on - Compressible flow. - Mach wave and Mach angle. | CO1 | 5 5 |
|  | b. | Obtain the Area-velocity relation and also examine the results of it. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Derive the governing equations for the flow through normal shock with suitable assumptions. | CO2 | 20 |
| 3. | a. | Derive Rayleigh pitot formula for supersonic flow. | CO2 | 10 |
|  | b. | Describe the shock wave generation and the types of shock wave with suitable sketches. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Discuss the expansion hodograph and mention its salient features. | CO2 | 20 |
| 5. |  | Discuss the flow characteristics through convergent-divergent duct with neat sketches. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Consider the air flows through a pipe of inside diameter =0.15m and length =30m.The inlet flow conditions are M1=0.3, P1=1bar, T1=273K. Assuming f=const=0.005,calculate the flow conditions at the exit,M2,P2,T2,P02. | CO1 | 20 |
| 7. |  | Write short notes on - Supercritical airfoil - Tip effects | CO3 | 10 10 |
| (OR) |  |  |  |  |
| 8. |  | Explain the effects of wing profile and aspect ratio in transonic flow. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | The pressure and temperature and the Mach number at the entry of a flow passage are 2 bar, 275 K and 1.3 respectively. If exit Mach number is 2.4 determine; a. Velocity of sound at stagnation conditions b. The maximum velocity c. Temperature and pressure at exit d. Critical Mach number at entrance and exit, Take  = 1.3 and R = 0.46 kJ/kgK. | CO1 | 5 5 5 5 |

ALL THE BEST