Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14BT2003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PRINCIPLES OF CHEMICAL ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Cracked gas from a petroleum refinery has the following composition by volume methane 45%,ethane 10%,ethylene 25%,propane 7%,propylene 8% and n-butane 5%. Find (a).the average molecular weight of the gas mixture (b).the composition by weight (c).specific gravity of the gas mixture | CO 1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | In a double effect evaporator plant, the second effect is maintained under vacuum of 500 torr. Find the absolute pressure in kpa,bar and psi | CO1 | 10 |
|  | b. | What is the average molecular weight of air and its composition by weight? | CO1 | 6 |
|  | c. | Unit conversion: 1m3 = …..………….. lt , 1 bar = ……………. pa 1 ft2 = ….…………………… in2 1 torr = …………..………… mm Hg | CO1 | 4 |
| 3. | a. | A bottle contains 5 kg water and 5 kg sodium hydroxide. What are the mass fraction and mole fraction of each component in the bottle? | CO1 | 10 |
|  | b. | Dilute ethyl alcohol is concentrated using a distillation column. The feed is 1,000 kg/hr of dilute alcohol with 25% alcohol by weight. The bottom product contains 0.4% alcohol by weight. The product contains 94% alcohol by weight. Find the percentage loss of water to the total water in the feed | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. |  | A multiple effect evaporator system has a capacity of processing10,000 kg/day of solid caustic soda. When it concentrates weak liquor from 4 to 25% by weight caustic soda. When the same plant is fed with 10% weak liquor and if it is concentrated to 50% (both in weight basis).find the capacity of the plant in terms of solid caustic soda? Assume that the water evaporating capacity to be same in both cases. | CO2 | 20 |
| 5. | a. | When liquid benzene is completely burned to carbon dioxide and liquid water, the standard heat of combustion is – 3267.6 kJ/mol. The standard heat of combustion of hydrogen to liquid water is – 285.83 kJ/mol and that of carbon to carbon dioxide gas is – 393.51 kJ/mol. Calculate the standard heat of formation of liquid benzene. | CO2 | 15 |
|  | b. | Calculate the heat that must be removed in cooling 32 kg of oxygen from 488K to 313K using Cp° data: Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) Gas; a; b x 103; c x 106; d x 109. Oxygen; 26.0257; 11.7551; - 2.3426; - 0.5623. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Define: Limiting reactant | CO3 | 2 |
|  | b. | Dryer system handles 500 kg/day of wet solids. Wet solids containing 80% solids and the product coming out has 2% moisture. Calculate the percentage of original water that is removed in the product. | CO3 | 10 |
|  | c. | Formulae of Heat of content in terms of formation | CO3 | 3 |
|  | d. | Formulae of Heat of reaction | CO3 | 3 |
|  | e. | Define: Excess reactant | CO3 | 2 |
| 7. | a. | Flue gases leaving the boiler stack at 523K have the following composition: CO2 = 111.31%,H2O = 13.04%,O2 = 2.17% and N2 = 73.48% (by volume). Calculate the heat lost in 1 kmol of gas mixture above 298K using the heat capacity data given below: Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) Gas; a; b x 103; c x 106; d x 109. CO2; 21.3655; 64.2841; - 41.0506; 9.7999. H2O; 32.4921; 0.0796; 13.2107; - 4.5474. O2; 26.0257; 11.7551; - 2.3426; - 0.5623. N2; 29.5909; - 5.141; 13.1829; - 4.968. | CO3 | 15 |
|  | b. | Calculate the heat of formation of gaseous n-heptane at 298.15 K using the following data: Data: Standard heat of formation of CO2= - 393.51 kJ/mol Standard heat of formation of H2O = - 285.83 kJ/mol Standard heat of combustion of C7H16 = - 4853.43 kJ/mol | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. |  | The gases leaving the boiler at 523K have the following composition: CO2 = 11.31%, H2O=2.17%,O2= 13.04% and N2=73.48% by volume. Calculate the heat lost in 1 kmol of gas mixture above 298K,using the heat capacity data given below: Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) Gas; a; b x 103; c x 106; d x 109. SO2; 21.3655; 64.2841; - 41.0506; 9.7999. SO3; 32.4921; 0.0796; 13.2107; - 4.5474. O2; 26.0257; 11.7551; - 2.3426; - 0.5623. N2; 29.5909; - 5.141; 13.1829; - 4.968. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Calculate the heat of formation of gaseous ethyl alcohol at 298.15K using the following data: Standard heat of formation of carbon dioxide = -393.51 kJ/mol Standard heat of formation of water = -285.83 kJ/mol Heat of combustion of gaseous ethyl alcohol at 298.15K = -1410.09kJ/mol | CO3 | 20 |

ALL THE BEST