Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14CE2002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF SOLIDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An aluminum bar, having cross sectional area of 1000 mm2, is subjected to axial forces as shown in fig 1. Fig.1 Find the elongation of the bar. Take E = 1.05 x 105 N/mm2. | CO1 | 8 |
|  | b. | Three bars made of copper, zinc and aluminum are of equal length and have cross section 500,750 and 1000 mm2 respectively. They are rigidly connected at their ends as shown in fig 2. Fig.2 If this compound member is subjected to a longitudinal pull of 250 kN, estimate the proportional of the load carried on each rod and the induced stresses. Take the value of for copper E = 1.3 x 105 N/mm2, for zinc E = 1.0 x 105 N/mm2, for aluminum E = 0.8 x 105 N/mm2. | CO2 | 12 |
| (OR) |  |  |  |  |
| 2. |  | A steel tube of 30 mm external diameter and 20 mm internal diameter encloses a copper rod of 15 mm diameter to which it is rigidly joined at each end. If, at a temperature of 10⁰C there is no longitudinal stress, Calculate the stresses in the rod and tube when the temperature is raised to 200⁰C. Take the value for steel and copper as E = 2.1 x 105 N/mm2 and 1 x 105 N/mm2 respectively. The value of coefficient of linear expansion for steel and copper is given as 11 x 10-6 per ⁰C and 18 x 10-6 per ⁰C respectively. | CO2 | 20 |
| 3. | a. | A small block is 4 cm long, 3 cm height and 0.5 cm thickness. It is subjected to uniformely distributed tensile forces of resultants 1200 N and 500 N as show in fig 3. Compute i) The normal stress ii) Shear stresses developed along the diagonal of AB. Fig.3 | CO2 | 12 |
|  | b. | A steel rod 5 m long and 30 mm diameter is subjected to an axial tensile load of 500 kN. Determine the change in length, diameter and volume of the rod. Take E = 2 x 105 N/mm2 and Poisson’s ratio 0.25. | CO2 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | A rectangular block of material is subjected to a tensile stress of 110 N/mm2 on one plane and a tensile stress of 47 N/mm2 on the plane at right angles to the former. Each of the above stresses is accompanied by a shear stress of 63 N/mm2 and that associated with the former tensile stress tends to rotate the block anticlockwise. Find i) Direction and Magnitude of each of the principal stress ii) Magnitude of maximum shear stress. | CO2 | 14 |
|  | b. | A rectangular bar of cross sectional area 10000 mm2 is subjected to an axial load of 20 kN. Determine the normal and shear stress on a section which is inclined at an angle of 30⁰ with normal cross section of the bar. | CO2 | 6 |
| 5. | a. | A cantilever 1.5 m long is loaded with a uniformely distributed load of 2 kN/m run over a length of 1.25 m from the free end. It also carries a point load of 3 kN at a distance of 0.25 m from the free end. Draw the shear force and bending moment diagrams of the cantilever. | CO3 | 10 |
|  | b. | A simply supported beam of length 10 m, carries the uniformely distributed load and two points loads as shown in fig 4. Draw the shear force and bending moment diagrams for the beam. Also calculate the maximum bending moment. Fig.4 | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Draw the S.F. and B.M. diagrams for the overhanging beam carrying uniformely distributed load of 2 kN/m over the entire length and a point load of 2 kN as shown in fig 5. Locate the point of contraflexure and also find maximum bending moment. Fig.5 | CO3 | 20 |
| 7. |  | A timber beam of reactangular section of length 8m is simply supported . The beam carries a UDL of 12 kN/m run over the entire length and a point load of 10 kN at 3 m from the left support. If the depth is two times the width and the stress in the timber is not exceed 8 N/mm2, Find the suitable dimensions of the section. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A solid cylindrical shaft is to transmit 300 kN power at 100 r.p.m. - If the shear stress is not to exceed 20 N/mm2, Find its diameter. - What a percentage saving in weight would be obtained if this shaft is replaced by a hollow shaft one whose internal diameter equal to 0.6 of the exteranal diameter, the length, material and maximum shear stress being the same? | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9 |  | A cantilever of length 2 m carries a uniformly distributed load 2 kN/m over a length of 1 m from the free end, and a point load of 1 kN at the free end as shown in fig 6. Fig.6 Determine, i) Slope at the free end. ii) deflection at the free end. Take E=2.1 x 105 N/mm2 and I=6.667 x 107 mm4. | CO4 | 20 |

ALL THE BEST