Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplemantary Examination – June – 2017

| Code : | 14CE2003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF FLUIDS | Max. marks : | 100 |

(For Mechanical Sciences)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the dynamic viscosity of oil, which is used for lubrication between a square plate of size 0.8m × 0.8m and an inclined plane with angle of inclination 30o. The weight of the square plate is 300N and it slides down the inclined plane with a uniform velocity of 0.3 m/s. The thickness of oil film is 1.5mm. | CO1 | 14 |
|  | b. | Calculate the density, specific weight and weight of 1 litre of petrol of specific gravity 0.7. | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. |  | A U-tube manometer is used to measure the pressure of water in a pipe line, which is in excess of atmospheric pressure. The right limb of the manometer contains mercury and is open to atmosphere. The contact between water and mercury is in the left limb. Determine the pressure of water in main line, if the difference in the level of mercury in the limbs of U-tubes is 10 cm and the free surface mercury is in level with the centre of the pipe. If the pressure of the water in the pipe line is reduced to 9810 N/m2, calculate the new difference in the level of mercury. Sketch the arrangements in both cases. | CO1 | 20 |
| 3. | a. | Derive the continuity equation for three dimensional Cartesian coordinates. | CO1 | 15 |
|  | b. | A fluid flow field is given by V = 4x3i-10x2yj+2tk. Calculate the velocity and acceleration of the fluid particle at the point (2, 1, 3) at t =1. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | In a two dimentional incompressible flow, the fluid velocity components are given by u = x-4y & v = -y-4x. Show that velocity potential exists and determine its form. Find also the stream function. | CO1 | 20 |
| 5. |  | Derive the Euler’s equation of motion and deduce the expression to Benoulli’s equation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | The water is flowing through a pipe having diameters 20 cm and 10 cm at sections 1 and 2 respectively. The rate of flow through pipe is 35 lit/s. The section 1 is 6m above the datumand section 2 is 4 m above the datum. If the pressure at section 1 is 39.24 N/cm2, find the intensity of pressure at section 2. | CO3 | 20 |
| 7. |  | Three pipes of diameters 400 mm, 200 mm and 300 mm and lengths 400 m, 200 m and 300 m respectively are connected in series. The difference in water surface levels in two tanks is 16 m. If coefficients of friction for these pipes is same and equal to 0.005 determine the rate of flow of water considering the following condtions, (i) Minor losses. (ii) Neglecting minor losses. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | An oil of specific gravity 0.7 if flowing through a pipe of diameter 300mm at the rate of 500 lit/s. Find the head lost due to friction and power required to maintain the flow for a length of 1000m. Take kinematic viscosity as 0.29 stokes. | CO3 | 15 |
|  | b. | Find the loss of head when a pipe of diameter 200 mm is suddenly enlarged to a diameter of 400 mm. the rate of flow of water through the pipe is 250 lit/s. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the principle of venturimeter with a neat sketch. | CO2 | 5 |
|  | b. | A horizontal venturimeter with inlet and throat diameters 30 cm and 15 cm respectively is used to measure the flow of water. The reading of differential manometer connected to the inlet and throat is 20 cm of mercury. Determine the rate of flow. Take Cd=0.98. | CO2 | 15 |

ALL THE BEST