Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14CE2009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | REINFORCED CONCRETE STRUCTURES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Design a simply supported slab of 7 x 3 m supported on 230mm thick walls. Assume live load as 3 kN/m2 and floor finish of 1kN/m2. Use M20 concrete and Fe415 steel. | CO2 | 18 |
|  | b. | Draw the stress strain curve for steel and mark the salient points. | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. |  | Elaborate the design philosophies highlighting the various limit states | CO1 | 20 |
| 3. | a. | Design a two way slab of size 5mx4m for a live load of 4000N/m2.The load due to floor finish is 1500N/m2.Take one short edge to be discontinuous. | CO2 | 18 |
|  | b. | Ast required for the positive mid span bending moments along the shorted and longer directions are 380mm2 and 320mm2 respectively. Determine the area of steel required for torsion when both the edges at the corners are discontinuous. | CO3 | 2 |
| (OR) |  |  |  |  |
| 4. | a. | Design a beam of span 4m supported on walls 230mm thick. The load on the beam is 15kN/m. Use M20 concrete and Fe415 steel. | CO2 | 18 |
|  | b. | Under what circumstances will you go for doubly reinforced sections? | CO3 | 2 |
| 5. | a. | A column has an unsupported length of 3m. Effective length factor is 0.85. Diameter of the column is 400mm and is subjected to a factored load of 1500kN. Design the reinforcement if it is provided with i) lateral ties ii) Spiral reinforcement. Use M20 concrete and Fe415. | CO2 | 16 |
|  | b. | Differentiate between short and long column. | CO3 | 2 |
|  | c. | What is the condition for eccentricity to design the column as axially loaded column? | CO3 | 2 |
| (OR) |  |  |  |  |
| 6. |  | Design a circular column of size 400mm with effective length of 3.5m to resist a factored load of Pu=1300kN and Mu=125kN.m. Use M20 concrete and Fe415 steel. | CO2 | 20 |
| 7. |  | An RC column 300 x 300mm in section carries an axial load of 1000kN. Design the footing for the column using M25 concrete and Fe415 steel. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Design the stem of a cantilever retaining wall to retain an earth embankment with a horizontal top 4m above ground level. The density of earth is 18kN/m3. Angle of internal friction  is 30o. The embankment is horizontal at its top. Safe bearing capacity of soil is 200kN/m2. Coefficient of friction between soil and concrete is 0.5. Adopt M 20 grade concrete and Fe415 HYSD bars. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a dog legged stair case for an office building in a room measuring 3mx6m (clear dimensions) Floor to floor height is 3.5m. The building is a public building liable to overcrowding. Stairs are supported on brick walls 230mm thick at the end of landings. M20 grade concrete and Fe 415 steel can be used. | CO2 | 20 |