Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14CE2010 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | STRUCTURAL ANALYSIS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Using the principle of virtual work, determine the vertical deflection of joint C of the pin jointed truss shown in figure, E = 200 x 106 KN/m2 and cross sectional area of each bar = 10 x 10-3 m2. | CO3 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Find the horizontal deflection at joint B in the frame ABCD shown in fig by using Unit Load method. EI is constant throughout. | CO3 | 20 |
| 3. |  | A simply supported beam has a span of 16 m is subjected to a UDL (dead load) of 13 kN/m (longer than the span) traveling from left to right. Draw the ILD for shear force and bending moment at a section 4 m from the left end. Use these diagrams to determine the maximum shear force and bending moment at this section. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Two point loads of 100 kN and 200 kN spaced 3 m apart cross a girder of span 12 m from left to right with the 100 kN leading. Draw the ILD for shear force and bending moment and find the values of maximum shear force and bending moment at a section 4 m from the left hand support. Also evaluate the absolute maximum bending moment due to the given loading system. | CO2 | 20 |
| 5. |  | Analyze the continuous beam ABC, shown in the figure and draw the BM diagram using Slope Deflection Method. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | ### Analyze the continuous beam ABC shown in figure by slope deflection method. The support B sinks by 15mm. Take E= 200 x 105 kN/m2 and I = 120 x 10-6 m4 | CO1 | 20 |
| 7. |  | Determine the support moments at A, B, C and D for the continuous girder shown in the figure by moment distribution method and draw the BMD. EI is the same for all members. | CO1 | 20 |
| (OR) |  |  |  |  |
| 8. |  | ### Analyze the continuous beam ABCDhaving flexural rigidity EI shown in figure by Moment distribution method. | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Use portal method to perform an approximate method of analysis of the frame given in figure. | CO3 | 20 |

ALL THE BEST