Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14CE2012 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FOUNDATION ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Describe the various methods of drilling holes for subsurface investigations with neat sketches. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Briefly explain about the various geophysical methods with neat sketches. | CO1 | 20 |
| 3. |  | Discuss about the different types of shallow and deep foundation with neat sketches? What are the factors governing the selection of foundation? | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Briefly explain about the standard penetration test. What are the various corrections in it? | CO2 | 20 |
| 5. | a. | Explain in detail about the different types of shear failure with neat sketch. | CO3 | 6 |
|  | b. | Determine the allowable gross load and net allowable load for a square footing of 2m side and with a depth of foundation 1m. Use Terzhagi’s theory and assume local shear failure. Take a factor of safety of 3. The soil at the site has angle of internal friction = 25°, unit weight of soil = 18kN/m3 and cohesion = 15kN/m2. | CO2 | 14 |
| (OR) |  |  |  |  |
| 6. |  | Derive the equation to find the bearing capacity of soil for strip footing by Terzhagi’s theory under general shear failure. Write the assumptions made in it. | CO2 | 20 |
| 7. |  | Discuss in detail about to determine the active earth pressure with neat sketches by - Rehbann’s method - Culmann’s method | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Design a strip footing which is placed at a distance of 1.5m and it is subjected to a load intensity of 350kN/m2. The thickness of the concrete wall is 400mm. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Determine the lateral earth pressure at rest per unit length of the wall shown in figure given below. Determine the location of the resultant earth pressure. Take Ko = 1-sinɸ,γw = 10kN/m3. | CO4 | 20 |