Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14CE3001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | APPLIED ELASTICITY AND PLASTICITY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the reason why Stress at a point is a second order tensor. | CO1 | 2 |
|  | b. | Explain body force and surface force with examples. | CO1 | 3 |
|  | c. | Develop Navier’s equations of equilibrium in 3D rectangular coordinates. | CO1 | 5 |
|  | d. | The state of stress at a particular point relative to xyz coordinate system is given by the following stress matrix 5 10 -10 10 10 0 MPa -10 0 40 Determine the normal stress and the magnitude and direction of the shear stress on a plane whose normal is equally inclined to the three coordinate axes. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the Principle of Superposition. | CO1 | 2 |
|  | b. | Explain the Generalized Hook’s law for an isotropic material. | CO1 | 3 |
|  | c. | Develop the St.Venant’s compatibility equations and explain their significance. | CO1 | 5 |
|  | d. | The displacement field in micro units for a body is given by U = (x2 + y) i + (3 + z)j + ( x + 2y)k Determine the principal strains at a point P (3, 1, -2) and their directions | CO1 | 10 |
| 3. | a. | Explain Plane stress problem with examples. | CO1 | 3 |
|  | b. | Develop the constitutive law for plane strain problem. | CO1 | 4 |
|  | c. | Develop the strain displacement relations in 2 D rectangular coordinates. | CO1 | 5 |
|  | d. | Explain Airy’s stress function and show that the following is a valid stress Function. φ=−Fd3xy2(3d−2y) Investigate what problem of plane stress is solved by it applied to the region bounded in y = 0, y = d, x = 0 on the side x+ve. | CO1 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Determine and sketch the stress field represented by asecond degree polynomial stress function. | CO1 | 3 |
|  | b. | Explain St.Venant Principle. | CO1 | 4 |
|  | c. | Develop the Biharmonic equation for plane strain problem. | CO1 | 5 |
|  | d. | Develop the following stress compatibility equation ▼2 ( σx + σy) = −(1+γ)δFxδx+δFyδy for plane stress problem. | CO1 | 8 |
| 5. | a. | For the stress function Alog r+ Br2 log r + Cr2 + D (where A, B, C and D are constants), compute the stresses in polar coordinates. | CO1 | 4 |
|  | b. | Develop the equilibrium equations in polar coordinates. | CO1 | 6 |
|  | c. | Determine the stresses in an infinite plate with a hole subjected to tension and calculate the stress concentration factor. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Develop the constitutive law for an axi-symmetric problem. | CO1 | 4 |
|  | b. | Develop the strain displacement relations in polar coordinates. | CO1 | 6 |
|  | c. | Develop the stresses in a curved bar under pure bending and compare them with the SOM solutions. | CO1 | 10 |
| 7. | a. | Explain Membrane Analogy for torsion of noncircular sections. | CO1 | 5 |
|  | b. | Develop the expression for maximum shear stress in a narrow thin rectangle subjected to torsion. | CO1 | 5 |
|  | c. | Develop the governing equation and boundary conditions in St. Venant’s warping function approach for torsion of noncircular sections. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Develop the expression for torsion of multi-celled thin walled tube. | CO1 | 5 |
|  | b. | A thin-walled member 1.2 m long has the cross-section wth uniform thickness 1mm shown in Fig. Determine the maximum torque which can be carried by the section if the angle of twist is limited to 10". What will be the maximum shear stress when this maximum torque is applied? For the material of the member G = 80 GN/m2. Radius 10 mm 40mm | CO1 | 5 |
|  | c. | Develop the expression for maximum shear stress and unit angle of twist for a Equilateral triangular shaft subjected to torque | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | What do you understand by yield criteria? | CO1 | 2 |
|  | b. | Distinguish between nominal stress, mean stress and true stress. | CO1 | 2 |
|  | c. | The State of stress at a point is given by σx = 70 MPa, σy = 120 MPa, τxy = 35 MPa and the yield strength for the material is 125 MPa, obtained from uniaxial tensile test. Determine whether yielding will occur according to Tresca’s or Von Mises yield conditions or not. | CO1 | 6 |
|  | d. | A thick cylinder of internal radius 15 cm and external radius 25 cm is subjected to an internal pressure” p” N/cm 2 . If the yield stress for the cylinder material is 220 N/mm2, determine the following : - i. The pressure at which the cylinder will start yielding just at the inner surface. ii. The stresses when the cylinder has a plastic front radius of 20cm. iii. The stresses when the entire cylinder has yielded Assume Von-Mises yield condition and a state of plane strain | CO1 | 10 |