Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EC2007 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | TRANSMISSION LINES & WAVEGUIDES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Sketch the equivalent circuit of a transmission line and give its significance. | CO1 | 5 |
|  | b. | Derive the primary and secondary constants of a transmission lines under lossless condition. | CO1 | 5 |
|  | c. | Briefly discuss about the various types of transmission lines used in practice. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A 300 m long line has the following primary constants: R= 4 KΩ, L = 0.5mH, G=60 mmho and C=20nF, operated at 12 MHz frequency. Find the secondary constants , velocity of propagation and wavelength. | CO1 | 15 |
|  | b. | Discuss the effects of loading on a transmission line. | CO2 | 5 |
| 3. |  | Examine how the standing waves are produced in a transmission line and hence derive the input impedance of a quarter wave and a half wave line. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A lossless transmission line with characteristic impedance Z0 = 50 Ω is terminated in a load of ZR = (75 +j50) Ω. Using Smith chart, find the following: i) Voltage Standing Wave Ratio ii) Magnitude and angle of reflection coefficient at the load iii) Magnitude and angle of reflection coefficient at the load iv) Source impedance at a distance of λ/8 from the load | CO2 | 20 |
| 5. |  | Derive the general solution of a parallel plate waveguide by applying suitable boundary conditions. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss about the wave properties in a parallel plate waveguide. | CO2 | 10 |
|  | b. | Derive the attenuation factor of a TEM wave in a parallel plate waveguide. | CO2 | 10 |
| 7. | a. | Identify the possible modes in a rectangular waveguide and discuss on its dominant mode. | CO2 | 10 |
|  | b. | Discuss about circular waveguide. | CO2 | 5 |
|  | c. | With neat sketches, explain how modes are excited in a rectangular waveguide. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Calculate the cut off frequencies for rectangular waveguide with the following inner dimensions. These operate in the dominant TE10 mode. (i) 7.214 x 3.404 cm(S-band) (ii) 2.286 x 1.106 cm(X-band) | CO2 | 10 |
|  | b. | A X-band waveguide with dimensions 2.286 x 1.016 cm has a cut off frequency of 6.56 GHz for the dominant mode. Calculate the phase velocity and group velocity at 8,10 and 12 GHz. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Relate the significance of a rectangular cavity resonator with a rectangular waveguide. | CO3 | 10 |
|  | b. | Brifly discuss about strip lines and slot lines. | CO3 | 10 |

ALL THE BEST