Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRIC CIRCUITS AND NETWORKS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Distinguish between the passive and active circuit elements. Discuss about various passive and active circuit elements. | CO1 | 10 |
|  | b. | Determine the supply voltage V. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Determine the equivalent resistant between the terminals A and B, | CO1 | 20 |
| 3. |  | Identify the mesh currents in the circuit shown below and determine those currents using mesh analysis method. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Given the network in Fig., find ZL for maximum power transfer and the maximum power transferred to the load | CO1 | 20 |
| 5. | a. | Obtain the condition for the optimum coupling in a single tuned coupled circuit. | CO1 | 10 |
|  | b | A 3 phase, balanced, star connected load of 5+j5 ohm is supplied from a balanced, three phase supply of 400V (line to line). Determine the phase and the line currents. Also determine the power consumed by the circuit. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the concept of series and parallel resonance. Also explain about the band width and quality factor of a circuit. | CO1 | 10 |
|  | b | Show that power in a 3 phase circuit can be measured using two wattmeters. | CO1 | 10 |
| 7. | a. | Derive an expression for the current in an R-L series circuit when DC voltage is applied. Also obtain the expression for the voltage across resistor and inductor. | CO2 | 10 |
|  | b. | The switch in the RC circuit shown in Fig. was closed at t = 0. There was no charge initially stored on the capacitor. Find vR, the voltage across the resistor at t = 60 s.Also determine the time at which vR is 2 V. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A series RL circuit with R = 100 and L = 20 h has a a.c. voltage of 200 volts applied through a switch at t = 0. Find (a) the equation for the transient current and voltage across the elements. (b) the current at t = 1 sec (c) the time at which eR=eL. | CO2 | 10 |
|  | b. | Derive an expression for the current in an R-C series circuit when DC voltage is applied.Also obtain the expression for the voltage across resistor and capacitor. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Determine the open circuit impedance parameters and short circuit admittance parameters of the π network given below. | CO1 | 10 |
|  | b. | Design a constant K low pass filter having a cut off frequency of 1.5 KHz to operate with a terminated load resistance of 600 ohm. | CO3 | 10 |

ALL THE BEST