Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE2009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRICAL MACHINE DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A 350 kW, 500 V, 450 rpm, 6 pole dc generator is built with an armature diameter of 0.87 m, and Core length of 0.32m, the lap wound armature has 660 conductors. Calculate specific electric and magnetic loading. | CO1 | 10 |
|  | b. | What are the factors influencing the choice of specific magnetic loading in electrical machines? | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Calculate the apparent flux density at a section of the teeth of an armature of a DC machine from the following data at that section: slot pitch = 24 mm, slot width = tooth width = 12 mm, length of armature core including 5 ducts of 10 mm each = 0.38, iron stacking factor = 0.92. True flux density in teeth at that section is 2.2 Wb/m2 for which the mmf is 70000 AT/m. | CO1 | 10 |
|  | b. | Obtain the relationship between real and apparent flux density in the slots of rotating machine. | CO1 | 10 |
| 3. | a. | Derive the expression for the power developed in DC machine in terms of its main dimensions. | CO2 | 10 |
|  | b. | Determine the main dimensions and number of poles of a dc generator with following data. 600kW, 900 rpm, 500V, specific magnetic loading 0.6 Tesla and specific electric loading 35000 ac/m, efficiency 91%, L/τ = 0.75assume square pole face. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | A shunt field coil has to develop an mmf of 9000 AT. The voltage drop in the coil is 40V, resistivity of the round wire is 0.021 Ω / m – mm2. The depth of winding is 35mm and length of mean turn is 1.4m. Design a coil so that the power dissipated is 700 W/m2 of the total coil surface. Take diameter of the insulated wire 0.2 mm greater than that of Bare wire. | CO2 | 20 |
| 5. |  | Determine the dimensions of core and yoke for a 200kVA, 50Hz, single phase, core type transformer. A cruciform core is used with distance between adjacent limbs equal to 1.6 times the width of core laminations. Assume voltage per turn 14V, maximum flux density 1.1 Wb/m2. Window space factor 0.32, current density 3 A/mm2, and stacking factor 0.9, the net iron area is 0.56d2 in a cruciform core where d is the diameter of circumscribing circle. Also width of largest stamp is 0.85d. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the expression for the number of cooling tubes required for the transformer. | CO3 | 10 |
|  | b. | Derive the output equation of single phase Transformer and three phase transformer. | CO3 | 10 |
| 7. |  | Estimate the main dimensions, airgap length, stator slots, stator turns per phase and cross sectional area of stator and rotor conductors for a three phase, 20HP, 400V, 4 pole, 50Hz, 1440 rpm induction motor. The motor is suitable for star delta starting. . Given that the average flux density in the air gap = 0.45 Wb/m2; Ampere Conductor per meter of armature periphery = 20000, full load efficiency = 0.9, full load power factor = 0.82 and kws = 0.955. Take L/τ = 0.85. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Design a cage rotor for a 40 HP, 3-phase, 400V, 50Hz, 6 pole, delta connected induction motor having a full load efficiency of 87% and a full load pf of 0.85 .Take D =33 cm and L=17 cm. Stator slots =54, conductors/slot= 14. Assume suitably any missing data if any. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Compute the main dimensions of a 1000 KVA, 50 Hz, three phase, 375 rpm alternator. The average air gap flux density is 0.55 Wb/m2 and ampere conductors per metre are 28000. Use rectangular poles. 'Assume the ratio of arc length to pole pitch equal to 2. Maximum permissible peripheral speed is 50 m/sec. The runaway speed is 1.8 times the synchronous speed. | CO2 | 20 |

ALL THE BEST