Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE2027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HVDC AND FACTS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Elucidate the technological development in control and protection, for better performance and reliability of dc transmission system. | CO1 | 10 |
|  | b. | Summarize the operating problems have surfaced that affected reliable operation and in few cases even lead to catastrophic failures of HVDC Technology. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss in detail the relative merits of the two modes of transmission which need to be considered by a system planner are based on economics of transmission, technical performance and reliability. | CO1 | 10 |
|  | b. | State the advantages and disadvantages of dc transmission system with following - Economics - Reliability - Performance. | CO1 | 10 |
| 3. | a. | Converse in detail the HVDC converter control characteristics, negative current margin and modified characteristics including VDCOL | CO2 | 10 |
|  | b. | Explain in detail the starting and stopping of a HVDC Link in terms of [i] Energisation and De-energisation of a bridge, [ii] Start-up of DC Link | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | A monopolar HVDC link has one bridge at each terminal. The parameters of the link are αmin = 5°, γmin = 18°, Rd = 5 ohms, Rcr=12 ohms, Vdor = 115kV, Iref at the rectifier = 1 kA, Iref at the inverter = 900 A, [i] If Vdoi = 117.5 kV, Calculate α, γ, Pi, and Qi [ii] Repeat [i] if Vdoi = 120 kV | CO3 | 10 |
|  | b. | Discuss with the block diagram of basic power and auxiliary controller including the generic model of VDCOL | CO2 | 10 |
| 5. | a. | Summarize the basics of power transmission networks with single line diagram and equivalent circuit. | CO1 | 10 |
|  | b. | Discuss about Control of Power Flow in AC Transmission Line. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explicate in detail about the classification of different FACTS controllers. | CO3 | 10 |
|  | b. | Illuminate the Shunt Compensation Connected at the Midpoint of the Line. | CO3 | 10 |
| 7. | a. | Describe the Principle of Operation of STATCOM. | CO1 | 10 |
|  | b. | Explain the operation of the SVC (FC+TCR) and derive the equations used. Also explain how the SVC is able to regulate the HVAC bus voltage. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Analyze a transmission line with SVC connected at midpoint with its Control characteristic. | CO2 | 10 |
|  | b. | Describe the different modes of operations of TCSC. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | The Unified Power Flow Controller (UPFC) proposed by Gyugyi is the most versatile FACTS controller – Discuss in detail. | CO3 | 10 |
|  | b. | Explain the operation of UPFC Connected at the Receiving End. | CO3 | 10 |

ALL THE BEST