Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE2033 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HARMONICS & POWER QUALITY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Categorize and illustrate the typical power quality phenomena defined in IEEE 1159. | CO1 | 10 |
|  | b. | Sketch the CBEMA & ITI curve and explain about events described in the curves. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Write down the basic steps involved in PQ Evaluation. | CO1 | 10 |
|  | b. | Examine the source and effects of different categories of Long duration voltage Variations. | CO2 | 10 |
| 3. | a. | In a three-phase system, the load voltage consists of Va=0.9sinωt, Vb=0.8sin(ωt-120), Vc=0.7sin(ωt+120). Calculate the following quantity with a sag period of 10ms, 20ms, 30ms with respect to ABC phases. i) Detroit Edison Sag Score (SS) ii) Voltage sag Energy iii) Voltage sag lost energy index (VSLEI) | CO2 | 08 |
|  | b. | Elucidate the different types of sag with a relevant vector diagram with proper equations. | CO2 | 12 |
| (OR) |  |  |  |  |
| 4. |  | Explain the following causes of sags. - Voltage sag due to motor sag - Voltage sag due to single line to ground fault. | CO3 | 20 |
| 5. |  | Converse the fundamental principle of over voltage protection of load equipments. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain in detail about surge arrestors and surge suppressors for over voltage Protection. | CO1 | 20 |
| 7. |  | Explain briefly about for the following harmonic filter. (i) Active filters (ii)Passive filters | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A single phase fully controlled bridge converter is fed from supply of 220V at 50Hz at a thyristors firing angle of α=300 Consider continuous load current of 20A. compute (a)THD of AC mains current (b) DIN (c) TDD of AC mains current (d) DF (e)DPF (f) PF | CO2/ CO3 | 12 |
|  | b. | Discriminate between linear and non linear load. | CO2 | 08 |
|  |  | Compulsory: |  |  |
| 9. | a. | Illustrate the principle of DVR operation used for sag mitigation. | CO1 | 10 |
|  | b. | Discuss the construction and working principle of UPQC. | CO2 | 10 |

ALL THE BEST