Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE2035 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | SWITCHED MODE POWER SUPPLIES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Compare and contrast a linear regulator with switched mode regulator. | CO1 | 8 |
|  | b. | Elaborate how the boost converter is ensured to operate in discontinuous conduction. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. |  | Explain the operation of a forward converter, its voltage conversion ratio and forward converter with transformer equivalent circuit. | CO1 | 20 |
| 3. |  | Design a full-bridge converter with the following specification: Vo = 5V, Po(max) = 50W, Io(max) = 10A, Io(min) = 1A, Vdc(max) = 60V, Vdc(min) = 38V, switching frequency = 50kHz. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Design a full-bridge converter with the following specification: Vo = 5V, Po(max) = 50W, Io(max) = 10A, Io(min) = 1A, Vdc(max) = 60V, Vdc(min) = 38V, switching frequency = 50kHz. | CO1 | 20 |
| 5. | a. | Explain how a MOSFET is selected based on its switching characteristics. | CO2 | 10 |
|  | b. | Why converters do required snubber circuit protection? Write the difference between the dissipative snubber and non-dissipative snubber circuit. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain how a power IGBT is selected based on its switching characteristics. | CO2 | 10 |
|  | b. | Analyze the turnoff loss of a transistor with snubber and without snubber circuit. | CO3 | 10 |
| 7. | a. | Explain the steps of a designing a transformer for a forward converter based on core and bobbin area (Ae,Ab), current density, frequency and peak flux density. | CO2 | 14 |
|  | b. | Brief why the core loss is more in a push pull converter. | CO2 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the steps of a designing a transformer for a push-pull converter based on core and bobbin area (Ae,Ab), current density, frequency and peak flux density. | CO2 | 14 |
|  | b. | Why ferrite core transformer is preferred for regulators? | CO2 | 6 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss about the various DC/AC inverter topologies used in the electronic ballast of fluorescent lamps. | CO2 | 20 |

ALL THE BEST