Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14EE3014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | WIND ENERGY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write a detailed note on the principal wind turbine components. | CO1 | 10 |
|  | b. | Highlight the desired properties of wind turbine materials. | CO2 | 5 |
|  | c. | A small 500 kW wind turbine with an initial cost of $500,000 is installed in Nebraska. The fixed cost ratio is 15% and the annual operation and maintenance (CO&M) is 2% of the initial cost. This system produces 65,0000 kWh/yr (AkWh). Determine the cost of energy (COE) for this system. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | A 40 m diameter, three bladed wind turbine produces 700 kW at a wind speed (hub height) of 14 m/s. The air density is 1.225 kg/m3. Find: - The rotational speed (rpm) of the rotor at a tip-speed ratio of 5.0. - What is the tip-speed (m/s)? - If the generator turns at 1800 rpm, what gear ratio is needed to match the rotor speed to the generator speed? What is the efficiency of the wind turbine system (including blades, transmission, shafts, and generator) under these conditions? | CO2 | 10 |
|  | b. | Discuss in detail with suitable examples the different wind turbine topologies. | CO2 | 10 |
| 3. | a. | The installation of a wind power project is a complex process involving a number of steps and legal and technical issues. Discuss in detail the various installation and operation issues of a wind power project. | CO1 | 10 |
|  | b. | Determine the wind speed at a height of 80 m over surface terrain with a few trees, if the wind speed at a height of 10 m is known to be 5 m/s. For your estimate use two different wind speed estimation methods (zo=100mm). Using the same methods determine the wind speed at 80 m if the trees were all removed from the terrain (zo=10mm). | CO1 | 5 |
|  | c. | Outline the significance of wind prediction and forecasting. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Analysis of time series data for a given site has yielded an average velocity of 6 m/s. It is determined that a Rayleigh wind speed distribution gives a good fit to the wind data. - Based on a Rayleigh wind speed distribution, estimate the number of hours that the wind speed will be between 9.5 and 10.5 m/s during the year. ii) Using a Rayleigh wind speed distribution, estimate the number of hours per year that the wind speed is equal to or above 16 m/s. | CO1 | 5 |
|  | b. | Many researchers have noted that a major barrier to deployment in many regions of the world is a lack of reliable and detailed wind resource data. Discuss in detail the different methodologies of wind resource assessment. | CO1 | 10 |
|  | c. | Write a short note on mechanics of wind motion. | CO2 | 5 |
| 5. | a. | The blades of a wind turbine are ready to be installed on a turbine on top of a ridge. The horizontal blades are supported at each end by saw horses, when a storm front arrives. The turbine crew huddles in their truck as the rain starts and the wind picks up, increasing eventually to 26.82 m/s (60 mph). Realizing that the wind coming up the western slope of the ridge roughly follows the 10 degree slope, the field engineer performs a quick calculation and drives his truck upwind of the blades to disrupt the airflow around the blades, preventing them from being lifted by the wind and damaged. The blades are 4.57 m (15 feet) long, 0.61 m (2 feet) wide, and have a mass of 45.36 kg (100 lbm). As the front arrives the temperature drops to 21.2°C (70°F). Assume that the blades are approximately symmetric airfoils (the engineer remembered that potential flow theory predicts that, prior to stall, the lift coefficient of a symmetric airfoil is approximately: Cl = 2π sinα ). Assume that the center of both the lift and the drag force is concentrated over the center of mass of the blade and that the leading edge is facing into the direction of the wind. Assume the air density is 1.20 kg/m3. i) Was there a reason to be concerned? At what wind speed will the blades be lifted by the wind, assuming that there is no drag? ii) If they are lifted by a 26.82 m/s (60 mph) wind, how fast will they be accelerated horizontally, if the blade's lift to drag ratio, Cd /Cl , is 0.03? | CO2 | 8 |
|  | b. | Obtain the value of Betz limit and determine the power from an ideal turbine rotor using the one dimensional momentum theory. | CO2 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Estimate the sound power level for a three-bladed, upwind wind turbine with the following specifications: rated power = 500 kW, rotor diameter = 40 m, rotor rotational speed = 40 rpm, wind speed = 12 m/s. | CO1 | 6 |
|  | b. | Derive the forces on a wind turbine blade and flow conditions at the blades using the momentum theory and the blade element theory. Obtain the expression for blade shape of an ideal rotor without wake rotation. | CO2 | 14 |
| 7. | a. | Discuss the general considerations for offshore wind energy, offshore wind turbines and their support structures. | CO1 | 10 |
|  | c. | The siting of a single turbine or a large-scale wind system for utility interconnection can be broken down into five major stages. What are they? Highlight the wind turbine siting issues, | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the operation of supervisory control and its implementation for wind turbines. | CO3 | 10 |
|  | b. | Consider a wind turbine generator connected to a grid system (see Figure) with a line-neutral system voltage, VS. The voltage at the wind turbine, VG, is not necessarily the same as VS. The distribution system resistance is R, and the distribution system reactance is X. The real generated power is P and the reactive power consumed by the wind turbine generator is Q. Suppose R = 8.8 Ω, X = 4.66 Ω, Vs = 11 kV, P = 313 kW, Q = 162 kVAr and the power factor at the generator is pf = 0.89. Determine the voltage difference between the grid and the generator, the voltage drop as a percent of the grid voltage, the fault level at the generator and the installed generator capacity as a percent of the fault level. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Review carefully the design issues and power system components of hybrid power system. | CO3 | 10 |
|  | b. | A wind turbine is connected to an 11 kV distribution line. The magnitude of the harmonic voltages, cn, at the point of common connection of the wind turbine and other grid users are listed in the following table: N; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. cn, volts; 11k; 120; 0; 85; 15; 0; 99; 151; 0; 12. N; 11; 12; 13; 14; 15. cn, volts; 216; 0; 236; 80; 0. What is the harmonic distortion of each harmonic and the total harmonic distortion? Are these within the allowable IEEE 519 limits? | CO3 | 10 |

ALL THE BEST