Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14FP2005 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HEAT AND MASS TRANSFER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A cold store is constructed from 20 cm thick concrete slabs, 20 cm thick insulation and 3 cm thick wooden cladding (thermal conductivities 1.10, 0.020 and 0.20 W m− 1K− 1, respectively). Calculate the rate of heat penetration to the cold store if the external air temperature is 10◦C and the internal air temperature is − 25◦C. Assume that the wall temperatures are at the respective air temperatures. | CO1 | 10 |
|  | b. | In an experiment to measure the thermal conductivity of meat, beef was formed into a square section block 5 cm × 5 cm and 1 cm thick. The edges of the block were insulated and heat was supplied continuously to one face of the block at a rate of 0.80 W. The temperatures of each face were measured with thermocouples and found to be 28.5 and 23.3◦C, respectively. What is the thermal conductivity of beef? | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive an expression for heat transfer in conduction through tubular pipe. | CO1 | 10 |
|  | b. | A cold storage wall (3 m X 6 m) is constructed of 15 cm thick concrete (thermal conductivity 1.37 W/m°C). Insulation must be provided to maintain a heat transfer rate through the wall at or below 500 W. If the thermal conductivity of the insulation is 0.04 W/(m°C), compute the required thickness of the insulation. The outside surface temperature of the wall is 38°C, and the inside wall temperature is 5°C. | CO2 | 10 |
| 3. | a. | Water flowing at a rate of 0.2 kg/s is heated from 30 to 70°C in a horizontal pipe (inside diameter 2.5 cm). The inside pipe surface temperature is 90°C. Estimate the convective heat-transfer coefficient if the pipe is 5 m long. | CO1 | 10 |
|  | b. | Explain the applications of dimensionless numbers in estimation of heat transfer coefficient. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Write in detail about convection heat transfer. | CO3 | 8 |
|  | b. | What is the expected percent increase in convective heat-transfer coefficient if the velocity of a fluid is doubled while all other parameters are kept the same for turbulent flow in a pipe? | CO2 | 8 |
|  | c. | Enumerate the steps for calculation of convective heat transfer coefficients using empirical correlations. | CO1 | 4 |
| 5. | a. | A pieceof meat carcase is kept in a deep freezer maintained at -18°C. Calculate the radiative heat transfer if the meat carcase is at 25°C and has an average area of 0.045m2 . The emissivity of carcase may be taken as 0.82. | CO1 | 8 |
|  | b. | Consider a 20 cm diameter spherical ball at 800 K suspended in air. Assuming the ball closely approximates a blackbody, determine (a) the total blackbody emissive power, (b) the total amount of radiation emitted by the ball in 5 min. | CO2 | 8 |
|  | c. | Write a note on application of radiation in food processing. | CO1 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | A continuous length of biscuit 0.20 m wide is baked in an oven which consists of a conveyor belt in a square section enclosure, the roof of which contains a radiative element 0.20 m wide. The biscuit is baked at 180°C and is conveyed directly beneath the element with a vertical separation of 0.40 m. The emissivities of element and biscuit are 0.95 and 0.80, respectively. If the rate of heat exchange is 900 W m− 1 , what will be the temperature of the element? | CO2 | 15 |
|  | b. | State Plank’s law and wien’s displacement law of radiation. | CO1 | 5 |
| 7. | a. | Derive an expression for LMTD. | CO2 | 10 |
|  | b. | Describe with a neat sketch the construction and working of scraped surface heat exchanger. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Explain with neat diagrams construction and working of the following: a. 1-2 pass shell and tube heat exchanger. b. Plate heat exchanger. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain in detail about film theory of mass transfer. | CO2 | 15 |
|  | b. | Write a note on convective mass transfer coefficient. | CO3 | 5 |

ALL THE BEST