Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14FP2016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PHYSICAL PROPERTIES OF FOOD MATERIALS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the sphericity of a cylindrical object of diameter 1.0 cm and height 1.7 cm. | CO1 | 8 |
|  | b. | Illustrate with a neat sketch the working of gas pycnometer. | CO2 | 8 |
|  | c. | The major diameter (L) and the average of the minor and major diameters (H) of barley are measured as 8.76 mm and 2.83 mm, respectively. Calculate the minimum and maximum radii of curvature for the barley. | CO3 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Cherry has a moisture content of 77.5% (wb). The apparent and bulk densities are 615 kg/m3 and 511 kg/m3 at 25◦C, respectively. Assuming cherry contains only carbohydrate and water, calculate the total porosity of cherries when stacked as bulk. The densities of carbohydrate and water are 1586 kg/ m3 and 997 kg/ m3, respectively. | CO1 | 4 |
|  | b. | Describe in detail about the volume determination through solid and liquid displacement method. | CO2 | 8 |
|  | c. | Explain the principle of measurement of radius of curvature. | CO3 | 8 |
| 3. | a. | Explain with a neat sketch the types of flow patterns available for flow of agricultural granular materials. | CO2 | 8 |
|  | b. | Describe the working of Farinograph and Amylograph in dough testing. |  | 8 |
|  | c. | List any 5 applications of viscosity in food engineering. | CO1 | 4 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the factors affecting the rheological parameters of food samples. | CO3 | 10 |
|  | b. | Describe with a neat sketch about the Texture Profile analysis. | CO2 | 10 |
| 5. | a. | Derive an expression for thermal conductivity of gases from the principles of Kinetic Theory of gases. | CO1 | 10 |
|  | b. | The composition of date fruit (Phoenix dactylifera) and the densities of food components are given below. Determine the thermal conductivity of the fruit at 25◦C, using parallel, series models. Component; Weight(%); Density (kg/m3). Water; 22.5; 995.7. Carbohydrate; 72.9; 1592.9. Protein; 2.2; 1319.6. Fat; 0.5; 917.15. Ash; 1.9; 2418.2. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the Radial heat flow methods available for determination thermal conductivity of foods. | CO2 | 10 |
|  | b. | Describe the construction and working of DSC. | CO1 | 10 |
| 7. | a. | Explain the following i. Henry’s law. ii. Raoult’s law. iii. Boiling point elevation. | CO2 | 10 |
|  | b. | Explain any one method of preparation of water activity sorption isotherm. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the boiling temperature of 10% (w/w) NaCl solution under atmospheric pressure. Data: Molecular weight of NaCl: 58.4 g/g-mole Enthalpy of saturated vapor: 2676.1 kJ/kg at 100◦C Enthalpy of saturated liquid: 419.04 kJ/kg at 100◦C R, gas constant: 8.31434 kJ/kg-mole K. | CO2 | 8 |
|  | b. | Describe with a neat sketch the Water Activity Determination by Hygrometer. | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Comparte dielectric constant and loss factor. | CO2 | 5 |
|  | b. | Summarize the basic principles of microwave heating. | CO3 | 5 |
|  | c. | Describe the effects of Composition of Foods on Dielectric Properties. | CO1 | 10 |

ALL THE BEST