Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENGINEERING MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A system of four forces acting on a body is shown in figure below. Determine the resultant force and its direction. 200 N 120 N 45 45 60o 40o 50N 100 N | CO1 | 14 |
|  | b. | Two spheres each of weight 1000 N and of radius 25 cm rest in a horizontal channel of width 90 cm as shown in the figure. Find the reactions on the points of contact A, B and C. | CO1 | 06 |
| (OR) |  |  |  |  |
| 2. | a. | Two cylinders of diameters 60mm and 30mm weighing 160N and 40N respectively are placed as shown in figure. Assuming all the contact surfaces to be smooth, find the reactions at A, B and C. A  | CO3 | 06 |
|  | b. | Five forces acting on a particle. The magnitude of the forces are 300 N, 600 N, 700N , 900 N and P and their respective angles with the horizontal are 0°, 60°, 135°, 210° & 270°. If the vertical component of the all forces is -1000 N. Find the value of P. Also calculate the magnitude and the direction of the resultant, assuming that the first force act towards the point, while all the remaining forces act away from the point. | CO1 | 14 |
| 3. | a. | A B 2m 2m 3m 3m 20kN 20kN/m 40kN/m Determine the reactions at support A and B of the overhanging beam loaded as shown in the diagram | CO3 | 15 |
|  | b. | Determine the moment about point A caused by 500 N. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Determine the reactions at supports A and B shown in Fig | CO1 | 15 |
|  | b. | Determine the moment about point A | CO1 | 5 |
| 5. | a. | Locate the Centroid of the Lamina | CO3 | 10 |
|  | b. | In the situation shown in the figure, determine the forces and the torque exerted by the screws on the beam. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Locate the centroid of the lamina. | CO3 | 20 |
| 7. | a. | A ladder rests against a smooth wall. There is no friction between the wall and the ladder, but there is a frictional force between the floor and the ladder. What is the minimum angle required between the ground and the ladder so that the ladder does not slip and falls to the ground? | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A man can pull horizontally with a force of 450n.a mass of 350 kg is resting on a horizontal surface for which the coefficient of friction is 0.20.The vertical cable crane is attached to the top of the block as shown. What will be the tension in the cable if the man is just able to start the block to the right? | CO3 | 14 |
|  | b. | A car initially moving at 45km/hr has to cover a distance of 7.6km in 8 minutes. With what acceleration, should it be speeded up so that it covers the distance exactly in time? | CO4 | 06 |
|  |  | Compulsory: |  |  |
| 9. | a. | A body of mass 15 kg is initially at rest on a inclined plane. Then it slides down. Calculate the distance moved by the body, on the inclined plane, when the velocity reaches to 6m/s.The coefficient of friction between the body and the plane is 0.1. | CO4 | 10 |
|  | b. | Two weights 80N and 20N are connected by a thread and move along a rough horizontal plane under the action of a force 40N, applied to the first weight of 80N as shown in fig. The coefficient of friction between the sliding surfaces of the weights and the plane is 0.3.determine the acceleration of the weights and the tension in the thread using D’Alemberts principle. | CO4 | 10 |