Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2017

| Code : | 14ME2014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENGINEERING THERMODYNAMICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State the Zeroth law of thermodynamics. | CO1 | 3 |
|  | b. | Energy – a property of the system. Illustrate. | CO1 | 5 |
|  | c. | Air flows steadily at the rate of 0.5 kg/s through an air compressor entering at 7 m/s velocity, 100 kPa pressure, 0.95 m3/kg volume and leaving at 5 m/s, 700 kPa, 0.19 m3/kg. The internal energy of the air leaving is 90 kJ/kg greater than the air entering. Cooling water in the compressor jackets absorb heat from the air at the rate of 58 kW. Compute (i) the rate of shaft work input to the air in kW (ii) ratio of the inlet pipe diameter to outlet pipe diameter. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Define thermodynamic cycle. | CO1 | 3 |
|  | b. | Differentiate path function and path function with suitable example. | CO1 | 5 |
|  | c. | Obtain the energy balance equation of a steady flow system and show that | CO1 | 12 |
| 3. | a. | Define thermal energy reservoir. | CO1 | 3 |
|  | b. | State the Clausius statement of the second law of thermodynamics. | CO1 | 5 |
|  | c. | Explain the process and working of a Carnot cycle with n eat sketch. | CO1 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | State the two reasons for irreversibility in any system. | CO2 | 3 |
|  | b. | Draw a cyclic heat engine and show the working of a refrigerator. | CO2 | 5 |
|  | c. | A domestic food freezer maintains a temperature of -15C. The ambient air temperature is 30C. If heat leaks into the freezer at the continuous rate of 1.75 kJ/s, what is the least power necessary to pump this heat out continuously. | CO2 | 12 |
| 5. | a. | Define a pure substance and state an example. | CO2 | 3 |
|  | b. | Draw the p-V diagram of a pure substance and denote the regions. | CO2 | 5 |
|  | c. | Explain the construcion and working of a throttling calorimeter. | CO2 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Define dryness fraction of pure substance. | CO3 | 3 |
|  | b. | Draw the p-v-T surface of a pure substance and denote the various regions. | CO3 | 5 |
|  | c. | A vessel of volume 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 250C. The mass of the liquid present is 9 kg. Find the pressure, mass, specific volume and enthalphy. | CO3 | 12 |
| 7. | a. | State Avagrado’s law. | CO3 | 3 |
|  | b. | Define compressibility factor. State the advantages of using generalized compressibility charts. | CO3 | 5 |
|  | c. | A certain gas has Cp = 1.968 and CV = 1.507 kJ/kg K. Find its molecular weight and gas constant. A constant volume chamber of 0.3 m3 capacity contains 2 kg of this gas at 5C. Heat is transferred to the gas until the temperature is 100C. Find the work done, heat transferred and change in internal energy. | CO3 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | Write the equation of state. | CO3 | 3 |
|  | b. | Explain Dalton’s law of partial pressures with illustration. | CO3 | 5 |
|  | c. | 0.1 m3 of hydrogen initially att 1.2 MPa, 200C undergoes a reversible isothermal expansion to 0.1 MPa. Find the (a) work done during the process (b) heat transferred (c) entropy change of the gas. | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define relative humidity. | CO4 | 3 |
|  | b. | Draw the sensible heating process and depict the same on a Psychrometric chart. | CO4 | 5 |
|  | c. | Explain the process and working of evaporative cooling with a neat sketch. | CO4 | 12 |

ALL THE BEST