Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2015 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMAL ENGINEERING - I | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The following readings are taken during the test on a boiler for onje hour.Steam generated =5400kg, calorific value of coal = 3150 kj/kg, coal burnt = 700kg, dryness fraction of steam entering superheater = 0.92, rated pressure of boiler = 11.5 bar, temperature of steam leaving superheater = 250°C, temperature of hot well = 45°C. Determine : a) Equivalent evaporation per kg of fuel with and without superheater b) thermal efficiency of the boiler with and without superheater c) Amount of heat supplied by the superheater per hour. | CO1 | 15 |
|  | b. | Write the difference between water and fire tube boiler? | CO1 | 3 |
|  | c. | What is the function of fusible plug ? | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. |  | The following observation were recorded during a boiler trial: Mass of feed water and its temperature =640 kg/hr and 50°C,Steam pressure =10 bar, Fuel used =55kg/hr,H.C.V. of fuel =44100 kj/kg. Temperature of gases =300°C,Boiler room temperature =30°C. For finding the dryness fraction of the steam, the steam is passed through the throttling calorimeter and the following observation are noted:Temperature of stream =120°C, Pressure of stream =5cm of Hg above atmosphere.Barometer reading =762mm of Hg,Heating surface of the boiler =18.6m2,Cpg (for gases) =1.1 kj/kg-k, Cps (superheated stream) =2.1kj/kg-k,The composition of oil used by mass, C =85%, H2=13% and remaining ash.The flue gas analysis by volume done by using Orsat apparatus is given as CO2=12.5%, O2 =4.5%, N2=83%. Find (a) Equivalent evaporation per kg of fuel and per m2 of heating surface area per hour.(b) Efficiency of boiler.(c) Also draw the heat balance sheet on one kg of fuel basis.Take the steam partial pressure in exhaust gases =0.7bar. | CO1 | 20 |
| 3. | a. | Estimate the mass flow rate of steam in a nozzle with the following data : Inlet pressure = 10bar, Inlet temperature =200°C, brake pressure =0.5bar, throat diameter =12mm | CO2 | 10 |
|  | b. | Dry saturated steam at 10 bar is expanded isentropically in a nozzle to 0.1bar.Find the dryness fraction of the steam at exit. Also find the velocity of steam leaving the nozzle when i) initial velocity is negligible ii) initial velocity of the steam is 135m/s | CO2 | 7 |
|  | c. | Define nozzle efficiency. | CO2 | 3 |
| (OR) |  |  |  |  |
| 4. |  | Derive an equation for the mass of steam discharged through nozzle and also the equation for the critical pressure ratio. | CO2 | 20 |
| 5. | a. | Draw the velocity triangles and derive the efficiencies. | CO3 | 15 |
|  | b. | Write the difference between impulse and reaction turbine? | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | The velocity of steam, leaving the nozzle of an implse turbine, is 1200m/s and the nozzle angle is 20°. The blade velocity is 375 m/s and the blade velocity coefficient is 0.75. Assuming no loss due to stock at inlet, calculation for a mass flow of 0.5 kg/s and symmetrical blading: (a) blade inlet angle; (b) driving force on the wheel; (c) axial thrust on the wheel; and (d) power developed by the turbine. | CO3 | 15 |
|  | b. | Elobrate the advantages of steam turbine over reciprocating steam engines | CO3 | 5 |
| 7. | a. | Determine the size of the cylinder for a double acting air compressor of 40kw indicated power,in which air in at 1 bar and 15°C and compressed according to the law pv1.2= constant, to 6 bar. The compressor runs at 100rpm with average piston speed of 152.5m/min. Neglect clearance | CO4 | 10 |
|  | b. | Define i) single acting compressor ii) Single stage compressor iii) Volumetric efficiency iv) Displacement of the compressor v) Free air delivery | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A single stage , single acting reciprocating air compressor has a bore of 200mm and a atoke of 300mm. It receives air at 1 bar and 20°C and delivers it at 5.5 bar.If the compression follows the law pv1.3= constant and clearance volume is 5 percent of the stroke volume, determine : i) The mean effective pressure ii) the power required to drive the compressor, if it runs at 500 rpm. | CO4 | 10 |
|  | b. | List the advantages of multi stage compression. | CO4 | 5 |
|  | c. | Estimate the maximum work required to compress 1 kg of air from 1 bar 27°C to 16 bar in two stages, if the law of compression is pv1.25= constant and the intercooling is perfect.Take R = 287J/kgK | CO4 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain briefly the principle of vapour compression refrigeration system. | CO5 | 15 |
|  | b. | Differentiate vapour compression and absorption systems. | CO5 | 5 |

ALL THE BEST