Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2017

| Code : | 14ME2016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMAL ENGINEERING II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Derive an expression for the air standard efficiency of dual cycle. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Derive an expression for the efficiency of a brayton cycle. Plot the cycle on pressure volume and temperature entropy diagram. | CO1 | 20 |
| 3. | a. | Compare two stoke and four stroke engines. | CO1 | 10 |
|  | b. | With e neat sketch explain the valve timing diagram of a four stroke diesel engine. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. |  | With neat sketch, explain the types of cooling system employed in I.C.Engines. | CO1 | 20 |
| 5. |  | With neat schematic, describe summer and winter air-conditioning systems. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A conference room for seating 100 persons is to be maintained at 22ºC dry bulb temperature and 60% relative humidity. The outdoor conditions are 40ºC dry bulb temperature and 27 º C wet bulb temperatures. The various loads in the auditorium are as follows: Sensible and latent heat loads per person 80 W and 50 W respectively; lights and fans 15000 W; sensible heat gain through glass, walls, ceilings etc. 15000 W. The air infiltration is 20 m3/min and fresh air supply is 100 m3/min. Two-third of recirculated air and one-third of fresh air are mixed before entering the cooling coil. The by-pass factor of the coil is 0.1. Determine room sensible heat factor and apparatus dew point. | CO4 | 20 |
| 7. |  | A conical diffuser has entry and exit diameters of 15 cm and 30 cm respectively. The pressure, temperature and velocity of air at entry are 0.89 bar, 340 K and 180 m/s. Determine the exit pressure, exit velocity and the force exerted on the diffuser walls | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Derive an expression for the area ratio as a function of Mach number. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the working principle of a turbo jet engine. Also, list out its advantages and disadvantages. | CO3 | 20 |

ALL THE BEST