Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2019 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HEAT AND MASS TRANSFER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A furnace wall is made up of three layers, inside layer with thermal conductivity 8.5 W/mK, the middle layer with conductivity 0.25 W/mK, the outer layer with conductivity 0.08 W/mK. The respective thickness of the inner, middle and outer layers are 25cm, 5cm and 3cm respectively. The inside and outside wall temperatures are 600 oC and 50 oC repectively. Draw the equivalent electrical circuit for conduction of heat through the wall and find thermal resistance, heat flow/m2 and interface temperatures. | CO1 | 15 |
|  | b. | Explain overall heat transfer coefficient. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain critical radius of insulation with a sketch showing variation in heat transfer with respect to radius of insulation. | CO1 | 5 |
|  | b. | A plane wall 10 cm thick generates heat at the rate of 4 x 104 W/m3 when an electric current is passed through it. The convective heat transfer co-efficient between each face of the wall and the ambient air is 50 W/m2K. Determine the surface temperature and the maximum temperature in the wall.Assume the ambient air temperature to be 20oC and thermal conductivity of the wall material to be 15W/m K. | CO1 | 15 |
| 3. | a. | A two-dimensional metal block is maintained at a constant temperature of 500oC at its upper face and 100oC at its lower face. The left face and the right face are both maintained at 100oC. Considering four interior nodal labeled as 1,2,3 and 4 inside the metal block, determine the temperatures at those interior nodal points. | CO1 | 10 |
|  | b. | A copper rod of outer diameter 20mm initially at a temperature of 380 oC is suddenly immersed in water at 100 oC.Determine the time required for the rod to reach 210 oC.Take convective heat transfer co-efficient as 95 W/m2 K. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | An aluminium slab of 5 cm thick is initially at a temperature of 400o C. It is suddenly immersed in water at 90oC. Calculate the mid plane temperature after 1 minute and also calculate the temperature inside the plate at a distance of 10 mm from the mid plane. Take h = 1800 W/m2K. | CO1 | 15 |
|  | b. | A sphere 300mm in diameter at 450 oC is buried under the earth at a depth of 2m from ground.Estimate the heat loss. Assume K soil=1 W/mK and earth temperature as 30 oC. | CO1 | 5 |
| 5. | a. | A vertical plate of 0.7 m width and 1.2 m height is maintained at a temperature of 90oC in a room at 30oC. Calculate the convective heat loss. | CO1 | 10 |
|  | b. | Find the heat loss from a rod 3mm in diameter and infinitely long when its base is maintained at 140oC.The conductivity of the material is 150 W/m K and the heat transfer co-efficient on the surface of the rod is 300W/m2 K.The temperature of the air surrounding the rod is 15oC. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Show by dimensional analysis that data for forced convection may be correlated by an equation of the form Nu=φ (Re,Pr). | CO1 | 20 |
| 7. | a. | A parallel flow heat exchanger is used to cool 4.2 kg/min of hot liquid of specific heat 3.5 kJ/kg K at 130oC. A cooling water of specific heat 4.18 kJ/kg K is used for cooling purpose at a temperature of 15oC. The mass flow rate of cooling water is 17 kg/min. Calculate (a) Outlet temperature of liquid (b) Outlet temperature of water (c) Effectiveness of heat exchanger.Take Overall heat transfer co-efficient is 1100 W/m2K.Heat exchanger area is 0.3m2. | CO2 | 15 |
|  | b. | Briefly explain the phenomenon of fouling in heat exchangers. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the diffusion rate of water from the bottom of a test tube of 25mm diameter and 35mm long into dry air at 25 oC.Take the diffusion co-efficient of water in air is 0.28x10-4 m2/s. | CO3 | 10 |
|  | b. | Helium diffuses through a plane membrane of 2 mm thickness. At the inner side the concentration of helium is 0.025 kg mole/m3. At the outer side the concentration of helium is 0.007 kg mole/m3. What is the diffusion flux of helium through the membrane. Assume diffusion co-efficient of helium with respect to plastic as 1 x 10-9 m2/s. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Assuming sun to be black body emitting radiation at 6000 K at a mean distance of 12 x 1010 m from the earth. The diamter of the sun is 1.5 x 109 m and that of the earth is 13.2 x 106 m. Calculate (a) total energy emitted by the sun (b) the emission received per m2 just outside the earth’s atmosphere (c) the total energy received by the earth if no radiation is blocked by the earth’s atmosphere (d) the energy received by a 2 x 2 m solar collector whose normal is inclined at 45o to the sun. The energy loss through the atmosphere is 50% and the diffuse radiation is 20% of direct radiation. | CO1 | 15 |
|  | b | Write a short note on radiation shield. | CO1 | 5 |

ALL THE BEST