Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2026 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF MACHINES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What are the various types of Kinematic Pairs? | CO1 | 7 |
|  | b. | In a four bar chain ABCD, AD is fixed and is 150 mm long. The crank AB is 40 mm long and rotates at 120 r.p.m. clockwise, while the link CD = 80 mm oscillates about D. BC and AD are of equal length. Find the angular velocity of link CD when angle BAD = 60 degree. | CO2 | 13 |
| (OR) |  |  |  |  |
| 2. | a. | What are the Inversions of Four Bar Chain? | CO1 | 10 |
|  | b. | Explain Quick Return Motion mechanism. | CO2 | 10 |
| 3. | a. | What are the types of Cams? | CO3 | 6 |
|  | b. | Design a cam for operating the exhaust valve of an oil engine. It is required to give equal uniform acceleration and retardation during opening and closing of the valve each of which corresponds to 60 degree of cam rotation. The valve must remain in the fully open position for 20 degree of cam rotation. The lift of the valve is 37.5 mm and the least radius of the cam is 40 mm. The follower is provided with a roller of radius 20 mm and its line of stroke passes through the axis of the cam. | CO3 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Explain any four terms used in Radial Cams. | CO3 | 5 |
|  | b. | A cam is to give the following motion to a knife-edge follower: (i). Outstroke during 60 degree of cam rotation. (ii). Dwell for the next 30 degree of cam rotation; (iii). Return stroke during next 60 degree of cam rotation, and (iv). Dwell for the remaining 210 degree of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. | CO3 | 15 |
| 5. | a. | What are the advantages and disadvantages of Gear Drive? | CO4 | 8 |
|  | b. | A pinion having 30 teeth drives a gear having 80 teeth. The profile of the gears is involute with 20 degree pressure angle, 12 mm module and 10 mm addentum. Find the length of path of contact, arc of contact and the contact ratio. | CO4 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Explain Compound Gear Train with a diagram. | CO4 | 8 |
|  | b. | In a reverted epicyclic gear train, the arm A carries two gears B and C and a compound gear D-E. The gear B meshes with gear E and the gear C meshes with gear D. The number of teeth on gears B,C and D are 75, 30 and 90 respectively. Find the speed and direction of gear C when gear B is fixed and the arm A makes 100 r.p.m. clockwise. | CO4 | 12 |
| 7. | a. | What are the types of friction? | CO5 | 8 |
|  | b. | A 150 mm diameter valve, against which a steam pressure of 2 MN/m^2 is acting, is closed by means of a square threaded screw 50 mm in external diameter with 6 mm pitch. If the coefficient of friction is 0.12; find the torque to turn the handle. | CO5 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | What are the laws of kinematic friction? | CO5 | 6 |
|  | b. | Determine the maximum, minimum and average pressure in plate clutch when the axial force is 4 kN. The inside radius of the contact surface is 50 mm and the outside radius is 100 mm. Assume uniform wear. | CO5 | 14 |
|  |  | Compulsory: |  |  |
| 9. | a. | What are the important factors upon which the selection of belt drive depends? | CO5 | 8 |
|  | b. | Find the power transmitted by a belt running over a pulley of 600 mm diameter at 200 r.p.m. The coefficient of friction between the belt and the pulley is 0.25, angle of lap 160 degree and maximum tension in the belt is 2500 N. | CO5 | 12 |

ALL THE BEST