Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2017

| Code : | 14ME2027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DYNAMICS OF MACHINERY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Questions | Course outcome | Marks |
| --- | --- | --- | --- |
| 1. | A horizontal steam engine running at 120 r.p.m. has a bore of 250 mm and a stroke of 400 mm. The connecting rod is 0.6 m and mass of the reciprocating parts is 60 kg. When the crank has turned through an angle of 45° from the inner dead centre, the steam pressure on the cover end side is 550 kN/m2 and that on the crank end side is 70 kN/m2. Considering the diameter of the piston rod equal to 50 mm, determine: - Turning moment on the crank shaft, - Thrust on the bearings, and - Acceleration of the flywheel, if the power of the engine is 20 kW, mass of the flywheel 60 kg and radius of gyration 0.6 m. | CO1 | 20 |
| (OR) |  |  |  |
| 2. | Calculate the required value T and various forces in links to maintain the equilibrium of the four bar mechanism shown below: | CO1 | 20 |
| 3. | A multi-cylinder engine is to run at a speed of 600 r.p.m. On drawing the turning moment diagram to a scale of 1 mm = 250 N-m and 1 mm = 3o, the areas above and below the mean torque line in mm2 are: + 160, – 172, + 168, – 191, + 197, – 162. The speed is to be kept within ± 1% of the mean speed of the engine. Calculate the necessary moment of inertia of the flywheel. Determine the suitable dimensions of a rectangular flywheel rim if the breadth is twice its thickness. The density of the cast iron is 7250 kg/m3 and its hoop stress is 6 MPa. Assume that the rim contributes 92% of the flywheel effect. | CO2 | 20 |
| (OR) |  |  |  |
| 4. | A shaft carries four masses A, B, C and D of magnitude 200 kg, 300 kg, 400 kg and 200 kg respectively and revolving at radii 80 mm, 70 mm, 60 mm, and 80 mm in planes measured from A at 300 mm, 400 mm and 700 mm. The angles between the cranks measured anticlockwise are A to B 45°, B to C 70° and C to D 120°. The balancing masses are to be placed in planes X and Y. The distance between the planes A and X is 100 mm, between X and Y is 400 mm and between Y and D is 200 mm. If the balancing masses revolve at a radius of 100 mm, find their magnitude and angular position. | CO3 | 20 |
| 5. | A vibrating system consists of a mass of 70 kg, a spring stiffness 40 kN/m and a damper. The damping provided is only 25% of the critical value. Calculate a) Damping factor, b) Critical damping coefficient, c) Natural frequency of damped vibrations, d) Logarithmic decrement, and e) Ratio of two consecutive amplitudes. | CO4 | 20 |
| (OR) |  |  |  |
| 6. | A machine has a mass of 100 kg and unbalanced reciprocating parts of mass 2 kg which mve through a vertical stroke of 80 mm with simple harmonic motion. The machine is mounted on four springs, symmetrically arranged with respect to centre of mass in such a way that the machine has one degree of freedom and can undergo vertical displacements only. Neglecting damping, calculate the combined stiffness of the spring in order that the force transmitted to the foundation is 1/25th of the applied force, when the speed of rotation of machine crank shaft is 100 r.p.m. When the machine is actually supported on the springs, it is found that the damping reduces the amplitude of successive free vibrations by 25%. Find: - The force transmitted to foundations at 1000 r.p.m. - The force transmitted to the foundation at resonance, and - The amplitude of the forced vibration of the machine at resonance. | CO4 | 20 |
| 7. | A steel shaft 1.5 m long is 95 mm in diameter for the first 0.6 m of its length, 60 mm in diameter for the next 0.5 m of the length and 50 mm in diameter for the remaining 0.4 m length. The shaft carries two flywheels at two ends, the first having a mass of 900 kg and 0.85 m radius of gyration located at the 95 mm diameter end and the second having a mass of 700 kg and 0.5 m radius of gyration located at the other end. Determine the location of the node and the natural frequency of free torsional vibration of the system. The modulus of rigidity of shaft may be taken as 80 GN/m2. | CO4 | 20 |
| (OR) |  |  |  |
| 8. | Determine the natural frequencies of torsional oscillations for the following system. The system is a reciprocating IC engine coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel is of 60 mm diameter and 950 mm length. The shaft from the pinion to the pump is of 40 mm diameter and 300 mm length. The engine speed is 1/4th of the pump speed. - Moment of Inertia of the flywheel is 800 kg m2 - Moment of Inertia of the gearwheel is 15 kg m2 - Moment of Inertia of the pinion is 4kg m2 - Moment of Inertia of the pump is 17 kg m2 The modulus of rigidity of the shaft material is 84 GN/mm2. | CO4 | 20 |
| Compulsory: |  |  |  |
| 9. | A Porter governor has two balls each of mass 3 kg and a central load of mass 15 kg. The arems are all 200 mm long and pivoted to the axis. If the maximum and minimum radii of rotation of the balls are 160 mm and 120 mm respectively, find the minimum and maximum speeds and also the range of speed. | CO5 | 20 |