Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2029. | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DESIGN OF MACHINE ELEMENTS | Max. marks : | 100 |

NOTE: Use of approved Design Data book and sheets are permitted, missing data can be assumed suitably.

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Classify the types of designs with example. | CO1 | 3 |
|  | b. | Define factor of safety. List its importance in designing. | CO1 | 3 |
|  | c. | Explain in detail about the factors influencing machine design. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Write a short note on fatigue limit. | CO1 | 4 |
|  | b. | A cylindrical member is loaded as shown in Figure. A load of 5000 N acts in a direction parallel to the area of cross section of the member. Determine the maximum shear stress induced in the member. | CO2 | 16 |
| 3. | a. | Name the types of cyclic stresses. | CO1 | 4 |
|  | b. | Determine the diameter of a piston rod subjected to a total load having a cyclic fluctuation from 150 kN in compression to 25 kN in tension. The endurance limit is 360 MPa and yield strength is 400 MPa. Take Factor of safety as 1.5; Surface finish factor as 0.88; Stress concentration factor as 2.25 and load correction factor as 0.8 | CO2 | 16 |
| (OR) |  |  |  |  |
| 4. | a. | Write the types of springs with its application. | CO1 | 4 |
|  | b. | Design a helical compression spring to carry a load of 1.5 kN with a deflection of 40 mm. The value of spring index is 5. The maximum permissible shear stress for the spring wire is 400N/mm2 and Modulus of rigidity is 8 x 1010 N/m2. | CO2 | 16 |
| 5. | a. | Define critical speed of a shaft. List its importance in designing. | CO1 | 4 |
|  | b. | A solid shaft running at 1000 rpm transmits 20 kW. The shaft carries an armature weighing 1kN at its center. The ends of the shaft are supported by two bearings placed 1 m apart. The permissible shear stress is 45 N/mm2 and the bending stress is 70 N/mm2 for the material of the shaft. Assuming the shock and fatigue factors for bending and torsion as 2 and 1.5 respectively, determine a suitable diameter for the shaft. | CO2 | 16 |
| (OR) |  |  |  |  |
| 6. | a. | Write the application of flexible couplings. | CO1 | 2 |
|  | b. | Design and draw a neat sketch of rigid flange coupling, which is to transmit 20 kW at 1000 rpm from an electric motor to a compressor. The following permissible stresses may be used. (a) Shear stress for shaft and key = 40 MPa; (b) Crushing stress for bolt and key = 80 MPa; (c) Shear stress for the coupling = 15 MPa. | CO3 | 18 |
| 7. | a. | Distinguish between a cotter joint and a knuckle joint. | CO1 | 2 |
|  | b. | Design a cotter joint that is used to transmit a load of 6 kN. The permissible design stresses may be taken as 60 MPa in tension, 40 MPa in shear and 90 MPa in crushing. Also draw a neat sketch of the cotter joint. | CO3 | 18 |
| (OR) |  |  |  |  |
| 8. | a. | What are the types of crankshafts? | CO1 | 2 |
|  | b. | It is required to design a knuckle joint to connect two circular rods subjected to an axial tensile force of 50 kN. The rods are coaxial and a small amount of angular movement between their axes is permissible. Design the joint and specify the dimensions of its components. Select suitable materials for the parts. | CO3 | 18 |
|  |  | Compulsory: |  |  |
| 9. |  | A cast iron flywheel is to be designed for a single cylinder double acting steam engine, which delivers 150 kW at 80 rpm. The maximum fluctuation of energy per revolution is 10 % of work done per cycle. The total fluctuation of speed is 4% of mean speed. If the mean diameter of the flywheel rim is 2.4 m, determine the cross-sectional dimensions of the rim. Assume the density of cast iron as 7200 kg/m3; Allowable tensile stress as 16 N/mm2. The width of the rim is twice the thickness. Assume number of arms as 6. Draw a neat sketch of the flywheel. | CO3 | 20 |

ALL THE BEST