Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 14ME2046 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | METAL CUTTING THEORY AND PRACTICE | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate between orthogonal and oblique cutting. | CO1 | 5 |
|  | b. | Show the various forces acting in an orthogonal cutting system with a help of Merchant Circle and explain the importance of the angles involved in the diagram. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. |  | During an orthogonal cutting following data was observed. Cutting Force =120 kgf Feed Force = 30 kgf Rake Angle = 10 Degrees Feed/ Revolution = 0.2 mm Width of the Cut = 2 mm Chip Thickness 0.4 mm Cutting Speed =100 m/min Determine the - Chip Thickness Ratio - Shear Angle - Cutting Speed | CO2 | 20 |
| 3. |  | Illustrate the cutting tool geometry and angles with a neat sketch of a single point cutting tool. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Explain how cutting in orthogonal cutting is affected by the following factors - Rake angle - Cutting Speed - Feed - Depth of cut | CO2 | 20 |
| 5. | a. | Suggest the cutting fluids used for the following - Free cutting steel - Medium carbon steel - Stainless steel - Aluminium - Cast Iron - Copper | CO3 | 6 |
|  | b. | Describe any two experimental methods to determine the cutting tool temperature with simple diagrams | CO3 | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss the importance of the desirable properties of a cutting tool material with reference to machining conditions. | CO5 | 10 |
|  | b. | Describe the coding and identification system used for index-able inserts | CO2 | 10 |
| 7. | a. | Define tool life. | CO5 | 2 |
|  | b. | Explain the relationship between tool life and cutting speed. | CO5 | 6 |
|  | c. | Explain the effect of tool geometry and cutting variables on tool life. | CO5 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | A carbide tool while machining a mild steel work piece was found to have a life of 1 hour and 40 minutes when cutting at 50 meter per minute. Find the tool life if the tool is to operate at speed 30% higher than previous one. Also calculate the cutting speed if tool is required to have a life of 2 hours and 45 minutes. Assume Taylor’s exponent n=0.28. | CO5 | 15 |
|  | b. | State any five factors which come into play while evaluating machinability of any metal. | CO5 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the any two reasons for chatter due to self-excitation vibration in machining with simple sketches. | CO4 | 20 |