Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplemantary Examination  – June – 2017

| Code : | 14AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | INTRODUCTION TO AEROSPACE ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the contributions of the Wright brothers. | CO1 | 4 |
|  | b. | Explain the invention of Sir George Cayley. | CO1 | 4 |
|  | c. | Define standard atmosphere. Derive an expression for pressure and density ratio in the isothermal region of the standard atmosphere. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2 |  | Calculate the values of pressure, density and temperature for the standard atmosphere at an altitude of 14000 m. The standard sea level values are pressure = 101325 N/ m2, density = 1.2256 Kg/m3 and temperature = 288.16 K. The temperature lapse rate a = -0.0065 K/m. | CO1 | 20 |
| 3. | a. | Define camber and explain the effect of camber on cl-α curve . | CO2 | 10 |
|  | b. | With help of neat sketch explain the fuselage construction. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | What is Aileron? What is its use and how does it help in aircraft maneuver? | CO2 | 8 |
|  | b. | Define the term pressure coefficient and plot C p Vs x/c - along the chord direction at the selected angle of attack for a standard airfoil. | CO2 | 12 |
| 5. | a. | What is airframe? Explain with example the primary and secondary aircraft structure. | CO2 | 6 |
|  | b. | List the structural component of the aircraft wing and explain with neat sketch the functions associated with them. | CO2 | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Distinguish between monocoque and semi monocoque structure. | CO2 | 8 |
|  | b. | With help of neat sketch explain wing construction and functions of its components. | CO2 | 12 |
| 7. | a. | How a jet propulsion sytem works? | CO1 | 6 |
|  | b. | Describe the working of a jet engine. Depict the Various thermodynamic processes occurring in it on P-V diagram. | CO2 | 14 |
| (OR) |  |  |  |  |
| 8. | a. | Explain with a neat sketch the elements and working of a turboprop engine. | CO1 | 10 |
|  | b. | A jet propulsion unit with turbojet engine having a forward speed of 95 km/hr produces a12 kN of thrust and uses 35 kg of air /sec. Find the relative exit velocity, thrust power, the propulsive power, and propulsive efficiency. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | The effective exit velocity of a rocket is 3000m/s, the forward flight speed is 1500 m/s and propellant consumption is 70 kg/sec. Calculate thrust and specific impulse. | CO2 | 10 |
|  | b. | Derive the expression for specific impulse (Isp) for a rocket engine in terms of the temperatureof combustion chamber (To) and the molecular weight of the propellant. | CO2 | 10 |