UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June- 2017

| Subject Code: 14BT2049 |  | Time: 3 hours |
| --- | --- | --- |
| Subject Title: PROCESS EQUIPMENT DESIGN |  | Maximum Marks: 100 |

#### Answer ALL questions (5 x 20 = 100 Marks)

a)Any missing data may be suitable be assumed.

b).Chemical engineering  hand book  or any two  chemical engineering books may be permitted.

| Q. No. | Sub Div. | Questions | Marks |
| --- | --- | --- | --- |
| 1. | a. | Discuss about the plant layout of Ethylene production with significance. | 10 |
|  | b. | Describe about the Engineering material properties and give its application. | 10 |
| (OR) |  |  |  |
| 2. |  | What are the parameters required for location of plant layout? | 20 |
| 3. | a. | Describe the Design procedure of Hemispherical heads, Elepsoidal head and Toripherical head. | 10 |
|  | b. | Estimate the thickness required for parts from the vessel 12 m height, dia 1.5 m. The vessel operating pressure 14 bar absolute and at a temperature of 300 ºC. Material of construction will be plain carbon steel. The corrosion allowance 2mm should be used .J is a factor as 1, Design stress of ‘f’ as 85 N/ mm2. | 10 |
| (OR) |  |  |  |
| 4. |  | Design the pressure vessel in which reactants entering are 75% H3PO4 (wt 885 kg), saturated Na2CO3( wt 1860 kg) and density of H3PO4, Na2CO3 are 1573 and 1261 kg / m3.The vapour space required is 2.5 m3 L/D is 1.5 inside pressure 4 kg/ cm2.J is 0.85 allowable stress for 10 kgf/ mm2. The top is flat covers with a reduction geared stirred the vessel will have a feed inlet at its top and its product are collected at nozzle in bottom. A shallow dished head is used for the bottom closure whose knuckle radius 125 mm , crown radius 1640mm. | 20 |
| 5. | a. | Design the 1-1 shell and tube heat exchanger with neat sketch. | 10 |
|  | b. | Calculate the heat transfer area of 1-2 heat exchanger from the following data, inlet and outlet temperature of hot fluid are 423K and 353 K respectively. Inlet and outlet temperature of cold fluid are 303K and 318K respectively. Data given. Overall heat transfer coefficient is 4100 W/m2 K, Heat loss = 407kw and LMTD correction factor = 0.80. | 10 |
| (OR) |  |  |  |
| 6. |  | A single effect evaporator is fed with 5000 kg/ hr of solution containing 1% solution by weight. Feed temperature is 303K and is to be concentrated to a solution of 2% solute by weight. The evaporator is at atmospheric pressure (101.325 k pa) and area of evaporator is 69m2 .Saturated steam is supplied at 143.3 k pa as a heating medium. Calculate steam economy and overall heat transfer coefficient. Data. Enthalpy of feed at 303K = 125.79 k j/ kg, Enthalpy of vapor at 101.325 k pa= 2676.1 k j / kg, Enthalpy of saturated steam at 143.325 k pa= 2691.5 kJ/ kg, Enthalpy of product= 419.1 k j/kg, Enthalpy of saturated water at383K= 461.30 k j/ kg, Saturation temperature of steam= 383K Boiling point of solution = boiling point water= 373K | 20 |
| 7. |  | - A liquid of 0.3 moles of A and rest B is to be fed into the continuous fractional column to give top product of 0.9 mole fraction A and bottom product 0.95 mole fraction of B. Feed is liquid at 283K and boiling point of A and B are 353K and 383.5K respectively. Enthalpy of feed liquid at 283K is 165 KJ/ kg mole. Enthalpy of bottom at boiling point is 16425 KJ/ kgmole. Latent heat of feed is 30000 KJ/ Kgmol. The efficiency is 70%. Reflux ratio is 5:1.find the actual plate required? X; 1; 0.78; 0.58; 0.41; 0.25; 0.013; 0. Y; 1; 0.9; 0.77; 0.63; 0.45; 0.26; 0. b. Find the Location of feed plate and the ideal plates for total reflux ratio | 20 |
| 8. | a. | Describe with neat design of packed bed column. | 10 |
|  | b. | 5000kg/hr of SO2 – air mixture containing 5% by volume SO2 is to be scrubbed with 2,00,000 kg/hr of water in a packed tower. The exit concentration of SO2 is reduced to 0.15% .The tower operates at 1 atmosphere. The equilibrium relationship is given by Y = 30X.Where Y = moles of SO2/ moles of air, X = moles of SO2/ moles of water. If the packed height of the tower is 420 cm, estimate the height of transfer unit (HTU). | 10 |
|  |  | Compulsory: |  |
| 9. |  | Describe the batch reactor design procedure step by step with neat sketch and Give its merit and demerits. | 20 |