Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 15AE3009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FINITE ELEMENT ANALYSIS IN AEROSPACE APPLICATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. |  |  | Explain about the step by step procedure inolved solving the FEA problem. | CO1 | 20 |
| (OR) |  |  |  |  |  |
| 2. |  |  | Consider a bar as shown in figure. An axial load of 200 kN is applied at point P. Take A1 = 2400 mm2, E1 = 70 X 109 N/m2, A2 = 600 mm2, E2 = 200 X 109 N/m2. Calculate the following - The nodal displacement at point P. - Stress in each material. - Reaction force. | CO1 | 20 |
| 3. |  |  | Derive the Shape function for Triangular element in global coordinate system. | CO1 | 20 |
| (OR) |  |  |  |  |  |
| 4. |  |  | Derive the shape function of Bilinear rectangular element in Local co-ordinate system (s,t). | CO1 | 20 |
| 5. |  |  | Derive the shape function of nine node quadratic quadrilateral element in Natural coordinate system. | CO2 | 20 |
| (OR) |  |  |  |  |  |
| 6. |  |  | Derive the shape function of one dimensional Linear Quadratic element in Global coordinate system. | CO1 | 20 |
| 7. |  |  | For the two bar truss shown in figure, determine the displacements of node 1 and stress in element 1-3. | CO2 | 20 |
| (OR) |  |  |  |  |  |
| 8. |  |  | For the plane stress element shown in figure, the nodal displacements are U1 = 2.0 mm v1 = 1.0 mm U2 = 0.5 mm v2 = 0.0 mm U3 = 3.0 mm v3 = 1.0 mm Determine the element stresses σx,σy, τxy, σ1 and σ2 and the principal angle θP. Let E = 2.10 X 105 N/mm2, v = 0.25 and t = 10 mm. All co-ordinates are in millimeters. | CO2 | 20 |
|  |  | Compulsory: |  |  |  |
| 9. |  |  | A furnace wall is made up of three layers, inside layer with thermal conductivity 8.5 W/mK, the middle layer with conductivity of 0.25 W/mK, the outer layer with conductivity 0.08 W/mK. The respective thickness of inner, middle and outer layer are 25 cm, 5 cm and 3 cm respectively. The inside temperature of the wall is 6000C and outside of the wall is exposed to atmospheric air at 300C with heat transfer coefficient of 45 W/m2K. Determine the nodal temperatures. | CO2 | 20 |

ALL THE BEST