Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June – 2017

| Code : | 15CE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | IRRIGATION ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the methods of irrigation in detail with flowchart and sketches. | CO1 | 15 |
|  | b. | What are the benefits of irrigation? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain in detail the factors that affects water requirements of crop. | CO1 | 10 |
|  | b. | Define consumptive use of water. Explain the factors that affect the consumptive use of water. | CO1 | 10 |
| 3. | a. | Define runoff. List the factors affecting runoff. What are the methods used to compute runoff? | CO2 | 15 |
|  | b. | Delivery of 10 m3/s to a 32 ha farm is continued for 4 hours. The tail water is 0.27 m3/s. Soil probing after irrigation indicates that 30 cm of water has been stored in the root zone. Compute the Application Efficiency. | C03 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | What are the different forms of precipitation. Explain them. | CO2 | 10 |
|  | b. | Write a note on evapotranspiration with suitable sketch. | CO2 | 10 |
| 5. | a. | With the aid of flowchart, discuss the types of water wells. | CO1 | 15 |
|  | b. | Write the advantages and disadvantages of tube wells. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | What is canal lining? What are its advantages? Discuss the types of canal lining in detail. | CO3 | 15 |
|  | b. | Write short notes on interference of wells. | CO3 | 5 |
| 7. |  | Write about lacey’s theory and regime condition. Also write the step by step procedure adopted in designing a channel. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Design an irrigation channel to carry 50m3/sec of discharge. The channel is to be laid at a slope of 1 in 4000. The critical velocity ratio for the soil is 1.1. Use Kutter's rugorsity Coefficient as 0.023. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the function of spursor groynes in terms of river training works and classify them with neat skethes. | CO3 | 10 |
|  | b. | Explain water logging, its causes and the effects. | CO3 | 10 |

ALL THE BEST