Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14CE2003 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF FLUIDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. | Two large plane surface are 2.4 cm apart. The space between the surfaces is filled with oil. What force is required to drag a very thin plate of surface area 0.5 sq.meter between the two large plane surfaces at a speed of 0.6 m/s. If, i) The thin plate is in the middle of the two plane surfaces ii) The thin plate is at a distance of 0.8 cm from one of the plane surfaces? Take the dynamic viscosity of oil=8.10×10-1 Ns/m2. |  | CO1 | 14 |
|  | b. | State the Newton’s law of viscosity and also explain the differents types of fluids with suitable curves. |  | CO1 | 6 |
| (OR) |  |  |  |  |  |
| 2. | a. | An open tank contains water upto a depth of 2 m and above it an oil of specific gravity 0.9 for a depth of 1 m as shown in fig 1. Calculate the pressure intensity, (i) At the interface of two liquids (ii) At the bottom of tank. Fig.1 |  | CO1 | 8 |
|  | b. | A diffential manometer is connected at the two points A and B of two pipes as shown in fig 2. The pipe A contains a liquid of specific gravity=1.5 while pipe B contains a liquid os specific gravity =0.9. The pressures at A and B are 9.81N/cm2 and 17.65 N/cm2 respectively. Evaluate the difference in mercury level in the differential manometer. Fig.2 |  | CO2 | 12 |
| 3. | a. | A pipe 450 mm diameter branches int two pipes of diameter are 300 mm and 200 mm respectively. If the average velocity in 450 mm diameter pipe is 3 m/s. Calculate the Discharge through 450 mm diameter pipe and Velocity in 200 mm diameter pipe if the average velocity in 300 mm pipe is 2.5 m/s. |  | CO1 | 10 |
|  | b | Derive the continuity equation for three dimensional Cartesian coordinates. |  | CO2 | 10 |
| (OR) |  |  |  |  |  |
| 4. | a. | A fluid flow field is given by V = x2yi+y2zj – (2xyz+yz2)k prove that it is a case of possible steady incompressible flow. Calculate the velocity and acceleration at the point (2, 1, 3). |  | CO1 | 14 |
|  | b. | A stream function is given by ψ=5x-6y. Calculate the velocity components and also magnitude and direction of the resultult velocity at any point. |  | CO1 | 6 |
| 5. | a. | Derive the Euler’s equation of motion and deduce the expression to Benoulli’s equation. |  | CO2 | 14 |
|  | b. | List out and explain different types of fluid flows with neat sketches. |  | CO3 | 6 |
| (OR) |  |  |  |  |  |
| 6. | a. | A pipe of diameter 400 mm carries water at a velocity of 25 m/s. The pressure at the points A and B are given as 29.43 N/cm2 and 22.56 N/cm2 respectively while the datum head at A and B are 28 m and 30 m. Evaluate the head loss of head between A and B. |  | CO3 | 14 |
|  | b. | A pipe, through which water is flowing, having diameters 20 cm and 10 cm at cross-sections 1 and 2 respectively. The velocity of water at section 1 is given 4 m/s. Calculate the velocity head at sections 1 and 2 and also rate of discharge. |  | CO3 | 6 |
| 7. |  | Three pipes of diameters 300 mm, 200 mm and 400 mm and lengths 450 m, 255 m and 315 m respectively are connected in series. The difference in water surface levels in two tanks is 18 m. Determine the rate of flow of water if coefficients of friction are 0.0075, 0.0078 and 0.0072 respectively. Considering the following condtions, (i) Minor losses. (ii) Neglecting minor losses. |  | CO3 | 20 |
| (OR) |  |  |  |  |  |
| 8. |  | The rate of flow of water through a horizontal pipe is 0.25 m3/sec. The diameter of the pipe is suddenly enlarged from 200 mm to 400 mm. The pressure intensity in the smaller pipe is 11.772 N/cm2. Determine, - Loss of head due to sudden enlargement - Pressure intensity in the largepipe - Power lost due to enlargement. |  | CO3 | 20 |
|  |  | Compulsory: |  |  |  |
| 9. |  | A 30 cm x 15 cm venturimeter is provided in a vertical pipe line carrying oil of specific gravity 0.9, the flow being upwards. The difference in elevation of the throat section and entrance section of the venturimeter is 30 cm. The differential U tube mercury manometer shows a gauge deflection of 25 cm. Calculate, a) the discharge of oil. b) The pressure difference between the entrance section and the throat section. Take Cd=0.98 and specific gravity of mercury as 13.6. |  | CO2 | 20 |

ALL THE BEST