Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

|  |  | Semester : | 2016-17 EVEN |
| --- | --- | --- | --- |
| Code : | 14CE2003 | Duration : | 3hrs |
| Sub. Name : | Mechanics of Fluids | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(For Mechanical Students only)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The dynamic viscosity of an oil, used for lubrication between a shaft and sleeve is 6 poise. The shaft is of diameter 0.4 m and rotates at 190 r.p.m. Calculate the power lost in the bearing for a sleeve length of 90 mm. The thickness of the oil film is 1.5 mm. | CO1 | 12 |
|  | b. | Calculate the capillary rise in a glass tube of 2.5 mm diameter when immersed vertically in i) water and ii) mercury. Take surface tension σ = 0.0725 N/m for water and σ = 0.52 N/m for mercury in contact with air. The specific gravity for mercury is given as 13.6 and angle of contact is 130°. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | A rectangular plane surface is 2 m wide and 3 m deep. It lies in vertical plane in water. Determine the total pressure and position of centre of pressure on the plane surface when its upper edge is horizontal and i) coincides with water surface ii) 2.5 m below the free water surface | CO1 | 10 |
|  | b. | An inverted differential mercury manometer connected to two pipes carrying water. Manometer reading is shown in Fig. Find the pressure difference between two pipes. | CO1 | 10 |
| 3. | a. | Water flows through a pipe AB 1.2m diameter at 3 m/s and then passes through a pipe BC 1.5 m diameter. At C, the pipe branches. Branch CD is 0.8 m in diameter and carries one third of the flow in AB. The flow velocity in branch CE is 2.5m/s. Find the volume rate of flow in AB, the velocity in BC, the velocity in CD and the diameter of CE. | CO2 | 10 |
|  | b | A fluid flow field is given by V = 4x3i-10x2yj+2tk. Calculate the velocity and acceleration of the fluid particle at the point (2, 1, 3) at t =1. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | In a two dimentional incompressible flow, the fluid velocity components are given by u = x-4y & v = -y-4x. Show that velocity potential exists and determine its form. Find also the stream function. | CO2 | 20 |
| 5. | a. | Water is flowing through a pipe having diameters 300 mm and 200 mm at the bottom and upper end respectively. The intensity of pressure at the bottom end is 24.525 N/cm2 and the pressure at the upper end is 9.81 N/cm2. Determine the difference in datum head if the rate of flow through pipe is 40 lit/s. | CO2 | 14 |
|  | b. | Water is flowing through a pipe of 5 cm diameter under a pressure of 29.43 N/cm2 and with a mean velocity of 2 m/s. Find the total energy per unit weight of water at a crossection, which is 5 m above the datum line. | CO2 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | A 30 cm × 15 cm venturimeter is provided in a vertical pipe line carrying oil of specific gravity 0.9, the flow being upwards. the difference in elevation of the throat section and entrance section of the venturimeter is 30 cm. the differential U-tube mercury manometer shows a gauge deflection of 25 cm. calculate i) the discharge of oil and ii) the pressure difference between the entrance section and the throat section. Take the coefficient of meter as 0.98 and specific gravity of mercury as 13.6. | CO2 | 20 |
| 7. | a. | The difference in water surface levels in two tanks, which are connected by three pipes in series of lengths 300 m, 170 m and 210 m and of diameters 300 mm, 200 mm and 400 mm respectively is 12m. Determine the rate of flow of water if coefficient of friction are 0.005, 0.0052 and 0.0048 respectively, considering i) minor losses and ii) neglecting minor losses. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | An oil of specific gravity 0.7 is flowing through a pipe of diameter 300 mm at the rate of 500 lit/s. Find the head lost due to friction and power required to maintain the flow for a length of 1000 m.Take ν = 0.29 stokes. | CO3 | 10 |
|  | b. | Derive the expression for head lost due to friction in a pipe flow. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the principle of venturimeter with a neat sketch. Derive the expression for the rate of flow of fluid through it. | CO2 | 10 |
|  | b. | An orifice meter with orifice diameter 15cm is inserted in a pipe of 30 cm diameter. The pressure difference measured by mercury oil differential manometer on the two sides of the orifice meter gives a reading of 50 cm of mercury. Find the rate of low of oil of specific gravity 0.9 when the coefficient of discharge of the meter is 0.64. | CO2 | 10 |

ALL THE BEST

Course Outcome:

Students at the end of the course will be able to

1.	Understand the various types of flow and flow profiles

2.	Carry out flow measurements

3.	Design pipe networks