Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | INTRODUCTION TO AEROSPACE ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the contributions of the Wright brothers. | CO1 | 4 |
|  | b. | Distinguish between troposphere and stratosphere. | CO1 | 4 |
|  | c. | Define standard atmosphere. Derive an expression for pressure and density ratio i) in the isothermal region of the standard atmosphere ii) in the gradient region of the standard atmosphere. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the invention of Sir George Cayley. | CO1 | 5 |
|  | b. | Define geopotential altitude. | CO1 | 3 |
|  | c. | Calculate the values of pressure, density and temperature for the standard atmosphere at an altitude of 15000 m. The standard sea level values are pressure = 101325 N/ m2, density = 1.2256 Kg/m3 and temperature = 288.16 K. The temperature lapse rate a = -0.0065 K/m. | CO1 | 12 |
| 3. | a. | Define aspect ratio and explain the effect of aspect ratio on cl-α curve. | CO2 | 6 |
|  | b. | With help of neat sketch explain wing construction. | CO2 | 6 |
|  | c. | Experiment were conducted in a wind tunnel with a wind speed 50 km /hr on a flat plate of size 2 m long and 1 m wide. The density of air is 1.15 kg/m3. The coefficient of lift and drag are 0.75 and 0.15 respectively. Determine i) Lift force. ii) Drag force. | CO2 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | What is Aileron? What is its use and how does it help in aircraft maneuver? | CO2 | 6 |
|  | b. | With help of neat sketch explain fuselage construction. | CO2 | 6 |
|  | c. | Define the term pressure coefficient and plot C p Vs x/c - along the chord direction at the selected angle of attack for a standard airfoil. | CO2 | 8 |
| 5. | a. | What is airframe? Explain with example the primary and secondary aircraft structure. | CO2 | 8 |
|  | b. | List the structural component of the aircraft wing and explain with neat sketch the functions associated with them. | CO2 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Distinguish between monocoque and semi monocoque structure. | CO2 | 8 |
|  | b. | Explain the construction of fuselage and its predominant types with a neat sketch. Explain the functions of its components. | CO2 | 12 |
| 7. | a. | Explain with a neat sketch the elements and working of a turbojet engine. | CO1 | 8 |
|  | b. | Consider a turbojet powered airplane flying at a standard altitude of 12000 m at a velocity of 650 km/h. The turbojet engine itself has inlet and exit areas of 0.50 and 0.42 m2 respectively. The velocity and pressure of the exhaust gas at the exit are 460 m/s and 0.25 bar respectively. Calculate the thrust of the turbojet. The standard sea level values are pressure = 101325 N/ m2, density = 1.2256 Kg/m3 and temperature = 288.16 K, lapse rate = -0.0065 K/m | CO2 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | Describe the working of a jet engine. Depict the Various thermodynamic processes occurring in it on P-V diagram. | CO1 | 10 |
|  | b. | A jet propulsion unit with turbojet engine having a forward speed of 95 km/hr produces a12 kN of thrust and uses 35 kg of air /sec. Find the relative exit velocity, thrust power, the propulsive power, and propulsive efficiency. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the working principle of rocket with neat sketch. | CO2 | 8 |
|  | b. | Derive the expression for specific impulse (Isp) for a rocket engine in terms of the temperatureof combustion chamber (To) and the molecular weight of the propellant. | CO2 | 12 |

ALL THE BEST